PROJECT NAME

CONSTRUCTION SITE
RISK ASSESSMENT

Comprehensive General Risk Assessment for Construction Site Activities

| Document Information | Details |
| --- | --- |
| Resource Category | Risk Assessments |
| Document Type | Completed Practical Risk Assessment |
| Revision | Rev. 00 |
| Status | Reference Document |

Prepared as a practical HSE reference resource.

# 1. Document / Assessment Information

| Field | Details |
| --- | --- |
| Project / Site Name |  |
| Location |  |
| Client |  |
| Main Contractor |  |
| Subcontractor |  |
| Assessment Reference |  |
| Assessment Date |  |
| Review Date |  |
| Activity | General Construction Site Activities |
| Assessed By |  |
| Position |  |
| Reviewed By |  |
| Approved By |  |
| Persons at Risk | Construction workers, supervisors, engineers, operators, drivers, contractors, visitors and members of the public who may be exposed. |
| Revision | 00 |
| Document Status | Completed Reference Assessment |

# 2. Purpose

This assessment identifies common foreseeable hazards associated with general construction site activities, evaluates potential consequences and establishes practical controls to reduce risks to workers, contractors, visitors and other affected persons. It is a completed practical reference assessment and must be adapted to the actual project, work sequence, site conditions, equipment, workforce and applicable requirements.

# 3. Scope

The assessment covers general construction activities including site access, traffic and mobile plant, excavation, work at height, scaffolds and ladders, lifting and material handling, electrical work, temporary power, tools and machinery, hot work, confined spaces, concrete and formwork, rebar, housekeeping, dust and noise, chemicals, environmental conditions, welfare, emergency arrangements and simultaneous operations.

# 4. Persons at Risk

| No. | Person / Group | Potential Exposure |
| --- | --- | --- |
| 1 | Construction workers | Workplace hazards arising from construction activities and interfaces |
| 2 | Supervisors/engineers | Exposure during supervision, inspection and coordination |
| 3 | Plant and equipment operators | Collision, overturning, struck-by and equipment hazards |
| 4 | Drivers/delivery personnel | Traffic movement, loading/unloading and site interfaces |
| 5 | Subcontractors | Exposure to construction activities and simultaneous operations |
| 6 | Maintenance personnel | Mechanical, electrical and stored-energy hazards |
| 7 | Visitors | Unfamiliarity with site hazards and controlled areas |
| 8 | HSE/emergency personnel | Exposure while inspecting or responding to incidents |
| 9 | Public/third parties | Exposure at site boundaries, roads and access interfaces |
| 10 | Workers in adjacent activities | Interaction with nearby simultaneous work |

# 5. Construction Risk Control Principles

Construction activities should be planned so that hazards are eliminated or reduced before work starts. Controls should follow the hierarchy of controls, with priority given to elimination, substitution and engineering controls before administrative controls and PPE. Work must be supervised, communicated and reviewed whenever conditions change.

| Priority | Principle | Examples |
| --- | --- | --- |
| 1 | Eliminate | Remove unnecessary work at height, manual handling or hazardous exposure where practicable. |
| 2 | Substitute | Use safer materials, equipment or work methods where reasonably practicable. |
| 3 | Engineering Controls | Guarding, barriers, edge protection, segregation, ventilation and mechanical aids. |
| 4 | Administrative Controls | Permits, procedures, training, supervision, inspections, signage and scheduling. |
| 5 | PPE | Use task-appropriate PPE as the final layer of protection. |

# 6. General Site Control Requirements

| No. | Control Area | General Requirement |
| --- | --- | --- |
| 1 | Site Planning | Identify hazards and interfaces before work begins and coordinate activities. |
| 2 | Competency | Use competent workers, operators, supervisors and specialist personnel. |
| 3 | Access / Egress | Maintain safe, unobstructed and adequately controlled access and emergency routes. |
| 4 | Traffic Management | Separate pedestrians and vehicles and control reversing and delivery movements. |
| 5 | Work at Height | Provide suitable platforms, edge protection, access systems and fall prevention. |
| 6 | Excavation | Identify services, assess ground conditions, protect edges and prevent collapse. |
| 7 | Lifting | Use suitable equipment, competent personnel, planned lifts and exclusion zones. |
| 8 | Electrical Safety | Control temporary power, isolation, damaged cables, panels and electrical access. |
| 9 | Plant / Tools | Inspect equipment and prevent use of defective or unsuitable machinery. |
| 10 | Housekeeping | Keep work areas clean, organized and free from unnecessary obstructions. |
| 11 | Fire / Hot Work | Control ignition sources, combustible materials, gas cylinders and fire protection. |
| 12 | Chemical Safety | Identify hazardous substances, provide SDS information, storage and suitable PPE. |
| 13 | Dust / Noise | Assess exposure and implement engineering, administrative and PPE controls. |
| 14 | Weather | Monitor heat, wind, rain, dust storms and visibility and adjust work accordingly. |
| 15 | Emergency | Maintain emergency response, first aid, rescue, fire and communication arrangements. |

# 7. Activities and Areas Covered

This assessment is intended as a broad construction-site reference and may be applied to building, civil, infrastructure, fit-out and associated construction activities. It should be supplemented by task-specific risk assessments, method statements, permits, lifting plans, JSA/JHA documents and specialist assessments where required.

# 8. Risk Assessment Methodology

Risk is assessed using a 5 × 5 matrix. Risk score = Likelihood × Severity. Initial risk represents the risk before additional controls are implemented. Residual risk represents the expected risk after the listed further controls are implemented and verified.

| Likelihood | Description |
| --- | --- |
| 1 | Rare |
| 2 | Unlikely |
| 3 | Possible |
| 4 | Likely |
| 5 | Almost Certain |

| Severity | Description |
| --- | --- |
| 1 | Minor injury |
| 2 | First-aid injury |
| 3 | Medical treatment injury |
| 4 | Serious/major injury |
| 5 | Fatality / catastrophic consequence |

# 9. Risk Classification

| Score | Risk Level | General Action |
| --- | --- | --- |
| 1–4 | Low | Maintain controls and monitor. |
| 5–9 | Medium | Improve controls where reasonably practicable. |
| 10–16 | High | Management attention and additional controls required before proceeding. |
| 17–25 | Very High | Do not proceed until adequate controls reduce the risk. |

# 10. 5 × 5 Risk Matrix

| Likelihood ↓ / Severity → | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 Almost Certain | 5 | 10 | 15 | 20 | 25 |
| 4 Likely | 4 | 8 | 12 | 16 | 20 |
| 3 Possible | 3 | 6 | 9 | 12 | 15 |
| 2 Unlikely | 2 | 4 | 6 | 8 | 10 |
| 1 Rare | 1 | 2 | 3 | 4 | 5 |

# 11. Detailed Construction Site Risk Assessment

The following assessment addresses representative construction-site hazards. Risk ratings are reference values and must be verified against actual site conditions, work methods, workforce competency and project requirements.

| No. | Activity / Hazard | Hazard Description | Persons at Risk | Potential Consequences | Existing Control Measures | Initial L | Initial S | Initial Score | Initial Level | Further Controls | Residual L | Residual S | Residual Score | Residual Level | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Site access and uncontrolled entry | Unauthorized or uncontrolled access to active construction areas | Workers, visitors, public | Struck-by injury, falls, exposure to hazardous work | Site security and access control. | 4 | 4 | 16 | High | Provide controlled entry points, secure perimeter, visitor induction, access authorization and supervision; restrict hazardous areas. | 1 | 4 | 4 | Low | Site Manager / Security |
| 2 | Vehicle and pedestrian interface | People and vehicles share uncontrolled routes | Workers, drivers, public | Struck-by or crushing injury, fatality | Basic traffic arrangements. | 4 | 5 | 20 | Very High | Implement traffic management plan; segregate pedestrians and vehicles; provide barriers, crossings, speed control, lighting and trained banksmen. | 1 | 5 | 5 | Medium | Logistics Supervisor |
| 3 | Reversing vehicles / mobile plant | Operator has limited rear visibility | Workers, banksmen, drivers | Crushing, collision, fatality | Vehicle alarms and operator awareness. | 4 | 5 | 20 | Very High | Minimize reversing; use one trained banksman where required; establish exclusion zones; use cameras/sensors and controlled routes. | 1 | 5 | 5 | Medium | Plant Supervisor |
| 4 | Work at height | Personnel work near open edges or elevated platforms | Workers, supervisors | Fall from height, serious injury, fatality | Basic guardrails/PPE. | 4 | 5 | 20 | Very High | Use collective fall prevention first; provide compliant platforms, edge protection and safe access; inspect systems; use fall restraint/arrest only where appropriate. | 1 | 5 | 5 | Medium | Construction Supervisor |
| 5 | Unsafe scaffold | Scaffold incomplete, altered or not inspected | Workers, contractors | Fall, collapse, falling objects | Scaffold access provided. | 4 | 5 | 20 | Very High | Use competent scaffolders; provide compliant erection, tagging, inspection and handover; prevent unauthorized alteration; maintain safe access and edge protection. | 1 | 5 | 5 | Medium | Scaffold Supervisor |
| 6 | Ladder misuse | Unsafe ladder position, damaged ladder or overreaching | Workers | Fall, sprain, serious injury | Portable ladders available. | 4 | 4 | 16 | High | Use ladders only for suitable short-duration work; inspect before use; secure correctly; maintain three-point contact and avoid overreach. | 1 | 4 | 4 | Low | Supervisor |
| 7 | Excavation collapse | Unsupported excavation or unstable soil | Workers, plant operators | Burial, crushing, fatality | Excavations visually checked. | 4 | 5 | 20 | Very High | Assess ground conditions; use suitable shoring, benching or battering; provide safe access; inspect after changes/weather; keep spoil and plant away from edges. | 1 | 5 | 5 | Medium | Excavation Supervisor |
| 8 | Underground services | Excavation damages buried utility service | Workers, operators, public | Electrocution, gas release, flooding, fire, fatality | Service drawings available. | 4 | 5 | 20 | Very High | Use permit-to-dig; locate and mark services; use detection and controlled excavation; maintain required clearances and emergency arrangements. | 1 | 5 | 5 | Medium | Permit Holder / Engineer |
| 9 | Lifting and suspended loads | Load drops or swings during lifting operation | Workers, riggers, public | Crushing, struck-by injury, fatality | Lifting equipment and supervision. | 4 | 5 | 20 | Very High | Use approved lift plan, competent lifting team, inspected accessories, exclusion zones, correct rigging and controlled communication. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 10 | Electrical shock | Contact with energized equipment, cable or temporary supply | Electricians, workers | Electric shock, burns, fatality | Electrical installations provided. | 4 | 5 | 20 | Very High | De-energize and isolate where practicable; apply LOTO; test for dead; protect cables; restrict access; use suitable protection and competent electricians. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 11 | Temporary power / damaged cables | Improper connections or damaged temporary electrical equipment | Workers, electricians | Shock, fire, equipment damage | Routine visual checks. | 4 | 5 | 20 | Very High | Use approved temporary distribution; inspect/test protection; protect cables from water and traffic; remove damaged equipment from service. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 12 | Plant and machinery | Contact with moving machinery or unguarded components | Operators, workers | Crushing, amputation, entanglement, fatality | Machine guarding and operator controls. | 4 | 5 | 20 | Very High | Use competent operators; maintain guards; conduct pre-use inspections; isolate before maintenance; establish exclusion zones around operating plant. | 1 | 5 | 5 | Medium | Plant Supervisor |
| 13 | Hand and power tools | Defective or incorrectly used tools | Workers | Cuts, electric shock, eye injury, amputation | Tool inspections and PPE. | 4 | 4 | 16 | High | Inspect tools before use; use correct tool and guards; remove defective tools; provide training and task-specific PPE. | 1 | 4 | 4 | Low | Supervisor |
| 14 | Hot work | Welding, cutting or grinding creates ignition source | Workers, contractors | Fire, burns, explosion, smoke exposure | Fire extinguishers available. | 4 | 5 | 20 | Very High | Use hot-work permit where required; remove/protect combustibles; provide fire watch, suitable extinguishers, gas-cylinder controls and post-work monitoring. | 1 | 5 | 5 | Medium | Hot Work Supervisor / HSE |
| 15 | Fire and emergency access | Fire routes or emergency equipment obstructed | All site personnel | Delayed evacuation, injury, fatality | Emergency equipment installed. | 3 | 5 | 15 | High | Keep escape routes clear; inspect alarms, extinguishers and emergency lighting; maintain emergency signage and access; conduct drills. | 1 | 5 | 5 | Medium | HSE / Site Manager |
| 16 | Confined space | Entry into tank, chamber, pit or restricted space | Entrants, rescuers | Toxic exposure, oxygen deficiency, engulfment, fatality | Confined-space controls available. | 3 | 5 | 15 | High | Use permit system; atmospheric testing; isolation; ventilation; trained entrants/attendants; rescue plan and suitable equipment. | 1 | 5 | 5 | Medium | Confined Space Supervisor |
| 17 | Concrete and formwork | Unstable formwork, concrete pour or materials | Workers, formwork crew | Collapse, struck-by, crush injury | Formwork installed under supervision. | 4 | 5 | 20 | Very High | Use approved formwork design and inspection; verify supports/bracing; control pour sequence and loads; maintain exclusion zones. | 1 | 5 | 5 | Medium | Construction Engineer |
| 18 | Rebar / sharp materials | Exposed reinforcement or sharp edges | Workers, steel fixers | Puncture, laceration, impalement | Basic PPE. | 4 | 4 | 16 | High | Cap/protect exposed rebar where required; use safe storage and handling; maintain clear access; wear suitable PPE. | 1 | 4 | 4 | Low | Supervisor |
| 19 | Manual handling | Lifting, carrying or moving heavy/awkward materials | Workers | Musculoskeletal injury, strains, crush injury | Manual handling awareness. | 4 | 3 | 12 | High | Assess loads; use mechanical aids/team lifting; reduce load size; improve storage height and work positioning; train workers. | 1 | 3 | 3 | Low | Supervisor |
| 20 | Housekeeping | Waste, debris, cables or materials create hazards | Workers, visitors | Trips, falls, cuts, fire | Routine cleaning. | 4 | 3 | 12 | High | Maintain planned housekeeping; remove waste frequently; route/protect cables; stack materials safely; assign responsibility and inspect work areas. | 1 | 3 | 3 | Low | Area Supervisor |
| 21 | Dust / silica exposure | Cutting, grinding or excavation generates respirable dust | Workers, nearby personnel | Respiratory disease, eye irritation, reduced visibility | Dust masks and water used. | 4 | 4 | 16 | High | Use wet methods/local extraction where suitable; isolate dusty activities; maintain housekeeping; select suitable respiratory protection and monitor exposure. | 1 | 4 | 4 | Low | HSE / Supervisor |
| 22 | Noise exposure | High noise from plant, tools and construction activities | Workers, contractors | Hearing damage, communication failure | Hearing protection available. | 4 | 3 | 12 | High | Assess noise; use quieter equipment where practicable; isolate noisy work; limit exposure and provide suitable hearing protection. | 1 | 3 | 3 | Low | HSE / Supervisor |
| 23 | Chemical exposure | Workers handle cement, solvents, fuels, paints or other hazardous substances | Workers, stores personnel | Burns, dermatitis, inhalation, poisoning, fire | Chemical containers and PPE. | 4 | 4 | 16 | High | Maintain SDS and chemical register; label/store correctly; provide ventilation, spill controls and suitable PPE; train users. | 1 | 4 | 4 | Low | HSE / Storekeeper |
| 24 | Heat stress / severe weather | Work during high heat, dust storms, rain or strong winds | Outdoor workers, plant operators | Heat illness, dehydration, reduced concentration, falls | Drinking water and breaks. | 4 | 5 | 20 | Very High | Implement heat-stress plan; provide hydration, shaded rest, work/rest scheduling and monitoring; suspend work during unsafe weather conditions. | 1 | 5 | 5 | Medium | Site Manager / HSE |
| 25 | Simultaneous operations | Different trades work in overlapping areas without coordination | All site personnel | Collision, dropped objects, exposure to hazardous work | Daily coordination meetings. | 4 | 5 | 20 | Very High | Use work-front coordination, permit controls, exclusion zones, sequencing and daily briefings; stop conflicting activities when necessary. | 1 | 5 | 5 | Medium | Construction Manager |

Risk scores are calculated as Likelihood × Severity. Residual ratings assume the listed additional controls are implemented, verified and maintained.

# 12. Key Construction Safety Requirements

| No. | Requirement | Expected Practice |
| --- | --- | --- |
| 1 | Site Planning | Plan work sequences, interfaces, access, resources and emergency arrangements before work starts. |
| 2 | Competency | Use competent workers, operators, supervisors and specialist personnel. |
| 3 | Traffic Control | Separate people and vehicles and control reversing and deliveries. |
| 4 | Work at Height | Use collective fall prevention and safe access systems. |
| 5 | Excavation Safety | Identify services, assess ground and protect excavations against collapse. |
| 6 | Lifting Safety | Use suitable lifting equipment, inspected accessories and competent personnel. |
| 7 | Electrical Safety | Control isolation, temporary power, cables and unauthorized access. |
| 8 | Plant Safety | Inspect machinery, maintain guards and segregate operating zones. |
| 9 | Fire Prevention | Control hot work, combustibles, gas cylinders and ignition sources. |
| 10 | Chemical Safety | Maintain SDS, labeling, storage, spill controls and suitable PPE. |
| 11 | Housekeeping | Keep work areas clean, organized and free from trip and fire hazards. |
| 12 | Health Risks | Control dust, noise, heat, chemicals and other occupational exposures. |
| 13 | Emergency | Maintain first aid, fire, rescue, evacuation and communication arrangements. |
| 14 | Simultaneous Work | Coordinate trades and prevent conflicting or overlapping hazardous activities. |
| 15 | Inspection | Conduct routine workplace, equipment and control inspections. |
| 16 | Stop Work | Stop unsafe work and correct conditions before resuming. |

# 13. Site Inspection and Monitoring

Construction controls should be verified through routine inspections, supervisor monitoring, HSE inspections, equipment checks and worker observations. Defective equipment, unsafe conditions and uncontrolled hazards must be corrected promptly and escalated when necessary.

| Inspection / Monitoring Activity | Minimum Focus |
| --- | --- |
| Daily work-front inspection | Access, housekeeping, work interfaces, barriers, PPE and changing conditions. |
| Plant and equipment inspection | Guards, brakes, alarms, tires, controls, leaks, attachments and certification/status. |
| Work-at-height inspection | Scaffolds, platforms, ladders, edge protection and fall-protection systems. |
| Excavation inspection | Ground stability, access, shoring, edge protection, water and services. |
| Lifting inspection | Equipment, accessories, load path, exclusion zone and communication. |
| Electrical inspection | Temporary boards, cables, plugs, protection, earthing and access. |
| Emergency inspection | Fire equipment, first aid, alarms, routes, muster points and emergency access. |
| Housekeeping inspection | Waste, material storage, trip hazards, combustible materials and access routes. |

# 14. Monitoring and Review of the Risk Assessment

| Review Trigger | Required Action |
| --- | --- |
| Before work starts | Confirm controls, competent personnel, equipment and work sequence. |
| Change in work method | Stop and reassess hazards and controls before continuing. |
| New equipment / plant | Verify suitability, inspection status, operator competency and controls. |
| Change in site layout | Review traffic, access, emergency routes and work interfaces. |
| Weather change | Reassess heat, wind, rain, dust and visibility conditions. |
| New subcontractor / activity | Coordinate interfaces and brief affected personnel. |
| Incident / near miss | Investigate and revise relevant controls. |
| Repeated non-compliance | Escalate, strengthen controls and verify closeout. |
| Periodic review | Confirm the assessment remains suitable for the current construction phase. |

# 15. Emergency and Rescue Arrangements

Emergency arrangements must be suitable for the site size, activities and hazards. Workers must know how to raise an alarm, report an emergency, reach the muster point and obtain first aid. Rescue arrangements must be planned for foreseeable events such as falls, excavation collapse, electrical incidents, fire, confined-space emergencies and plant incidents.

| Emergency Element | Requirement |
| --- | --- |
| Emergency communication | Provide reliable means of raising alarms and contacting emergency responders. |
| First aid | Provide suitable first-aid resources and trained first-aiders for the site. |
| Fire response | Maintain suitable extinguishers, alarms, access and evacuation arrangements. |
| Evacuation | Clearly identify routes and muster points and keep them unobstructed. |
| Rescue | Provide task-specific rescue arrangements for height, excavation, confined space and other high-risk activities. |
| Utility emergency | Maintain procedures for electrical, gas, water or other service damage. |
| Post-incident | Provide medical response, reporting, investigation and corrective action. |

# 16. Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Project / Construction Manager | Provide resources and ensure site HSE requirements are implemented. |
| Construction Manager / Supervisors | Plan work, coordinate trades, monitor controls and stop unsafe activities. |
| HSE Team | Inspect, advise, monitor compliance, support risk assessment and verify corrective actions. |
| Engineers / Technical Personnel | Provide technical controls, approved methods and specialist input where required. |
| Plant / Lifting Supervisors | Control equipment, operators, lifting activities and exclusion zones. |
| Workers | Follow procedures, use controls and PPE, attend briefings and report hazards. |
| Subcontractors | Comply with project requirements and provide task-specific risk assessments and method statements. |
| Visitors | Follow site induction, access restrictions and instructions from site personnel. |

# 17. Worker Briefing / Acknowledgement

Personnel affected by the activities covered by this assessment should be briefed on applicable hazards, controls, emergency arrangements, access restrictions and stop-work requirements before starting work.

| No. | Name | Position | Company | Signature | Date |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

# 18. Review and Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |

# 19. Disclaimer

This document is a general completed reference risk assessment for construction-site activities. It is not a substitute for project-specific risk assessments, method statements, permits, lifting plans, excavation permits, electrical procedures, confined-space assessments, engineered designs or specialist assessments. A competent person must review and adapt the assessment to actual site conditions, work methods, equipment, workforce competency, client requirements and applicable requirements. Additional controls may be required where the actual risk is higher than represented in this reference document. Risk ratings must be verified before work starts and whenever conditions change.