PROJECT NAME

ELECTRICAL WORK
RISK ASSESSMENT

Comprehensive Risk Assessment for Electrical Installation, Maintenance and Testing

| Document Information | Details |
| --- | --- |
| Resource Category | Risk Assessments |
| Document Type | Completed Practical Risk Assessment |
| Revision | Rev. 00 |
| Status | Reference Document |

Prepared as a practical HSE reference resource.

# 1. Document / Assessment Information

| Field | Details |
| --- | --- |
| Project / Site Name |  |
| Location |  |
| Client |  |
| Main Contractor |  |
| Subcontractor |  |
| Assessment Reference |  |
| Assessment Date |  |
| Review Date |  |
| Activity | Electrical Work |
| Assessed By |  |
| Position |  |
| Reviewed By |  |
| Approved By |  |
| Persons at Risk | Electricians, technicians, workers, supervisors, contractors, operators, visitors and others who may be exposed |
| Revision | 00 |
| Document Status | Completed Reference Assessment |

# 2. Purpose

This assessment identifies foreseeable hazards associated with electrical installation, maintenance, testing, temporary electrical systems and electrical interfaces, evaluates the potential risks, and establishes practical controls to prevent electric shock, arc flash, burns, fire, equipment damage and other harm. It is a completed practical reference assessment and does not replace a project-specific electrical risk assessment, isolation procedure, permit, design, competent-person determination or applicable electrical requirements.

# 3. Scope

The assessment covers electrical installation and maintenance, distribution boards, temporary power, cables, portable tools, generators, testing and commissioning, isolation and lockout/tagout, energized work, electrical panels, underground and overhead electrical services, damaged cables, wet environments, extension leads, earthing and bonding, residual-current protection, battery systems, arc-flash exposure, work near electrical equipment, housekeeping and emergency response.

# 4. Persons at Risk

| No. | Person / Group | Potential Exposure |
| --- | --- | --- |
| 1 | Electricians / technicians | Shock, arc flash, burns, falls and electrical equipment hazards |
| 2 | General workers | Contact with energized equipment, cables or temporary power |
| 3 | Supervisors / engineers / HSE personnel | Exposure during inspection, testing or supervision |
| 4 | Plant / equipment operators | Electrical contact through equipment or temporary power systems |
| 5 | Contractors / subcontractors | Electrical exposure during interfaces and concurrent work |
| 6 | Visitors | Unauthorized access to electrical work areas |
| 7 | Utility personnel | Exposure during service isolation or interface work |
| 8 | Emergency responders | Electrical and fire exposure during incident response |
| 9 | Public / third parties | Exposure to temporary electrical installations or damaged services |
| 10 | Maintenance personnel | Unexpected energization and stored electrical energy |

# 5. Electrical Safety Control Principles

Electrical work should be planned to eliminate exposure to energized systems wherever reasonably practicable. The preferred approach is to isolate, lock out, tag out, verify dead and control access before work. Energized work should only be undertaken when justified, authorized and subject to a suitable task-specific assessment and competent-person controls. Electrical systems must be installed, inspected, tested and maintained by competent persons.

| Priority | Principle | Examples |
| --- | --- | --- |
| 1 | Eliminate exposure | De-energize and isolate before work whenever reasonably practicable. |
| 2 | Prevent contact | Use suitable insulation, enclosures, barriers, guarding, segregation and safe distances. |
| 3 | Control energy | Use isolation, lockout/tagout, discharge, earthing and verification procedures. |
| 4 | Protect people | Restrict access, use competent personnel, appropriate PPE and controlled work zones. |
| 5 | Minimize consequences | Maintain fire protection, emergency response, rescue and first-aid arrangements. |

# 6. General Control Requirements

| No. | Control Area | General Requirement |
| --- | --- | --- |
| 1 | Planning | Identify electrical hazards, circuits, equipment, interfaces and work method before starting. |
| 2 | Competency | Use authorized and competent electrical personnel for electrical work. |
| 3 | Isolation | Isolate and lock/tag sources before work whenever practicable. |
| 4 | Verification | Test for dead using suitable test equipment and verify isolation before touching conductors. |
| 5 | Backfeed | Identify alternative supplies, generators, UPS systems, capacitors and other sources. |
| 6 | Protection | Use suitable enclosures, barriers, covers, insulation and residual-current protection as applicable. |
| 7 | Earthing / Bonding | Provide and verify appropriate earthing and bonding arrangements. |
| 8 | Temporary Power | Inspect temporary distribution, cables, plugs, sockets and protection devices. |
| 9 | Cables | Protect cables from mechanical damage, water, heat, vehicles and sharp edges. |
| 10 | Wet Areas | Control electrical work in damp or wet locations using suitable equipment and protection. |
| 11 | Overhead / Underground Services | Identify services and maintain required clearances or isolation. |
| 12 | Arc Flash | Assess arc-flash exposure where applicable and control boundaries/PPE. |
| 13 | Tools / Test Equipment | Use suitable, inspected and correctly rated insulated tools and test equipment. |
| 14 | Fire | Control ignition sources and maintain suitable fire response arrangements. |
| 15 | Emergency | Maintain electrical rescue, first aid, fire and emergency isolation arrangements. |

# 7. Equipment and Systems Covered

The assessment considers distribution boards, switchboards, breakers, isolators, generators, temporary power systems, cables, extension leads, plugs, sockets, portable electrical tools, testing instruments, battery systems, UPS systems, lighting circuits, electrical panels, motors, control equipment, earthing/bonding and electrical protective devices. Equipment must be suitable, inspected and used within its rated limits.

# 8. Risk Assessment Methodology

Risk is assessed using a 5 × 5 matrix. Risk score = Likelihood × Severity. Initial risk represents exposure before additional controls; residual risk represents the expected risk after the listed additional controls are implemented and verified.

| Likelihood | Description |
| --- | --- |
| 1 | Rare |
| 2 | Unlikely |
| 3 | Possible |
| 4 | Likely |
| 5 | Almost Certain |

| Severity | Description |
| --- | --- |
| 1 | Minor injury |
| 2 | First-aid injury |
| 3 | Medical treatment injury |
| 4 | Serious / major injury |
| 5 | Fatality / catastrophic consequence |

# 9. Risk Classification

| Score | Risk Level | General Action |
| --- | --- | --- |
| 1–4 | Low | Maintain controls and monitor. |
| 5–9 | Medium | Improve controls where reasonably practicable. |
| 10–16 | High | Immediate management attention and additional controls required. |
| 17–25 | Very High | Work should not proceed until adequate controls reduce the risk. |

# 10. 5 × 5 Risk Matrix

| Likelihood ↓ / Severity → | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 Almost Certain | 5 | 10 | 15 | 20 | 25 |
| 4 Likely | 4 | 8 | 12 | 16 | 20 |
| 3 Possible | 3 | 6 | 9 | 12 | 15 |
| 2 Unlikely | 2 | 4 | 6 | 8 | 10 |
| 1 Rare | 1 | 2 | 3 | 4 | 5 |

# 11. Detailed Electrical Work Risk Assessment

The following assessment addresses representative electrical-work hazards. Risk ratings are reference values and must be verified against the actual electrical system, voltage, equipment, work method, environment and isolation arrangements before implementation.

| No. | Activity / Hazard | Hazard Description | Persons at Risk | Potential Consequences | Existing Control Measures | Initial L | Initial S | Initial Score | Initial Level | Further Controls | Residual L | Residual S | Residual Score | Residual Level | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Contact with energized conductors | Worker contacts exposed live conductor | Electricians, workers | Electric shock, burns, fatality | Electrical competency and basic PPE. | 4 | 5 | 20 | Very High | De-energize wherever practicable; isolate, lock/tag, test for dead and control access; use suitable barriers and insulated tools. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 2 | Unexpected energization | Circuit becomes energized during maintenance | Electricians, technicians | Shock, arc flash, fatality | General isolation procedure. | 4 | 5 | 20 | Very High | Apply formal LOTO; identify all sources; lock/tag isolation points; verify zero energy before work; control keys and authorized persons. | 1 | 5 | 5 | Medium | Authorized Person |
| 3 | Backfeed / alternative supply | Generator, UPS, PV, battery or other source energizes isolated circuit | Electricians, workers | Shock, arc flash, fatality | Circuit identification. | 3 | 5 | 15 | High | Identify all energy sources; isolate and secure each source; test at point of work; discharge stored energy where applicable. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 4 | Arc flash | Fault or switching event produces arc energy | Electricians, technicians | Severe burns, eye injury, hearing injury, fatality | General electrical PPE. | 4 | 5 | 20 | Very High | Assess arc-flash exposure; establish boundaries; use appropriate arc-rated PPE and insulated tools; de-energize whenever practicable. | 1 | 5 | 5 | Medium | Electrical Engineer / Supervisor |
| 5 | Damaged cable | Cuts, exposed conductors or damaged insulation | Workers, electricians | Shock, fire, short circuit | Visual inspections. | 4 | 5 | 20 | Very High | Remove damaged cables from service; protect cables from mechanical damage; repair/replace by competent personnel; inspect before re-energization. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 6 | Temporary electrical installation | Improper temporary distribution or connections | Workers, contractors | Shock, fire, equipment damage | Temporary boards and routine checks. | 4 | 5 | 20 | Very High | Use approved temporary distribution; provide suitable protective devices; inspect, test and maintain; keep connections protected and controlled. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 7 | Extension leads | Overloaded, damaged or poorly routed extension leads | Workers, electricians | Shock, trip, fire | General cable management. | 4 | 4 | 16 | High | Use suitable rated leads; inspect before use; avoid overloading; protect from vehicles/water/sharp edges; use appropriate RCD/RCCB protection. | 1 | 4 | 4 | Low | Supervisor |
| 8 | Wet / damp environment | Electrical equipment used where water or moisture is present | Workers, electricians | Electric shock, short circuit, fatality | General waterproofing. | 4 | 5 | 20 | Very High | Eliminate water exposure where possible; use equipment rated for environment; provide appropriate RCD protection; isolate before handling wet equipment. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 9 | Inadequate earthing / bonding | Equipment has ineffective protective earth | Workers, electricians | Shock, fire, protective device failure | Earthing provided in installation. | 3 | 5 | 15 | High | Verify continuity and earth arrangement; inspect bonding; test protective systems; rectify defects before use. | 1 | 5 | 5 | Medium | Electrical Engineer |
| 10 | Faulty protective device | RCD/MCB/fuse fails to operate correctly | Workers, electricians | Shock, fire, equipment damage | Protective devices installed. | 3 | 5 | 15 | High | Use correctly rated devices; test RCD/RCCB and protection systems as required; replace defective devices; maintain records. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 11 | Overloaded circuit | Circuit/load exceeds design capacity | Workers, electricians | Overheating, fire, equipment failure | Basic load control. | 3 | 4 | 12 | High | Verify circuit capacity and load; prevent unauthorized connections; provide correctly rated protection and distribution. | 1 | 4 | 4 | Low | Electrical Engineer |
| 12 | Electrical panel access | Unauthorized or unprotected access to live panel | Workers, visitors | Shock, arc flash, fatality | Panel covers and warning signs. | 4 | 5 | 20 | Very High | Keep panels secured; use appropriate barriers and warning signage; restrict access to authorized persons; maintain required clearances. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 13 | Testing / commissioning | Testing exposes workers to energized equipment | Electricians, technicians | Shock, arc flash, burns | Competent testers and test equipment. | 4 | 5 | 20 | Very High | Prepare test procedure; define energized boundaries; use suitable rated test equipment/PPE; control access; de-energize when possible. | 1 | 5 | 5 | Medium | Testing Engineer |
| 14 | Incorrect test equipment | Test instrument unsuitable, damaged or incorrectly rated | Electricians, technicians | Shock, arc flash, inaccurate result | Pre-use visual check. | 3 | 5 | 15 | High | Use correctly rated, calibrated and inspected equipment; verify function before/after testing using approved method; use suitable leads and probes. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 15 | Improper isolation point | Wrong circuit or isolator is identified | Electricians, workers | Unexpected energization, shock | Circuit labeling. | 4 | 5 | 20 | Very High | Confirm drawings and circuit identification; trace and verify isolation at point of work; use test-for-dead procedure and controlled authorization. | 1 | 5 | 5 | Medium | Authorized Person |
| 16 | Overhead electrical lines | Plant/personnel or tools approach overhead lines | Workers, operators, electricians | Electrocution, fire, fatality | General awareness. | 4 | 5 | 20 | Very High | Identify lines; maintain required clearance; isolate/de-energize where possible; establish exclusion zones and dedicated spotter where needed. | 1 | 5 | 5 | Medium | Site Manager / HSE |
| 17 | Underground electrical services | Excavation damages buried electrical cable | Workers, operators, public | Electrocution, arc flash, fatality | Service drawings and detection. | 4 | 5 | 20 | Very High | Locate and mark services; use permit and controlled excavation; maintain safe clearance; follow emergency isolation procedure. | 1 | 5 | 5 | Medium | Permit Holder / Competent Person |
| 18 | Portable electrical tools | Tool has damaged insulation, plug or cable | Workers | Shock, burns, fire | Pre-use checks. | 4 | 4 | 16 | High | Inspect before use; use suitable protection; remove defective tools from service; maintain inspection/testing records as required. | 1 | 4 | 4 | Low | Supervisor |
| 19 | Battery / stored energy | Battery, capacitor or UPS retains hazardous energy after isolation | Electricians, technicians | Shock, burns, arc flash | Isolation awareness. | 3 | 5 | 15 | High | Identify stored energy; isolate and discharge according to manufacturer procedure; verify zero energy before work; use appropriate PPE. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 20 | Fire from electrical fault | Short circuit, overload or poor connection causes fire | Workers, electricians, public | Burns, smoke inhalation, property damage, fatality | Fire extinguishers available. | 4 | 5 | 20 | Very High | Maintain sound installation, correct protection and housekeeping; control ignition sources; provide suitable electrical-fire response equipment and emergency procedures. | 1 | 5 | 5 | Medium | Electrical Supervisor / HSE |
| 21 | Poor housekeeping around electrical equipment | Combustible materials or water stored near electrical equipment | Workers, electricians | Fire, shock, restricted access | Routine housekeeping. | 3 | 4 | 12 | High | Maintain clear electrical working spaces; remove combustibles and water; keep access to isolation devices unobstructed. | 1 | 4 | 4 | Low | Supervisor |
| 22 | Electrical work at height | Electrician works from ladder/scaffold while exposed to electrical hazards | Electricians, workers | Fall combined with shock, fatality | Access equipment and PPE. | 4 | 5 | 20 | Very High | Control electrical energy first; provide suitable access; use stable platforms; prevent overreach and maintain tool control. | 1 | 5 | 5 | Medium | Electrical Supervisor |
| 23 | Confined electrical work area | Electrical work performed in restricted or conductive location | Electricians, technicians | Shock, difficult rescue, fatality | General confined-area controls. | 3 | 5 | 15 | High | Assess space-specific electrical hazards; isolate sources; use suitable low-voltage/approved equipment where applicable; maintain communication and rescue arrangements. | 1 | 5 | 5 | Medium | Electrical Supervisor / HSE |
| 24 | Unauthorized electrical modification | Unapproved connection or alteration to electrical system | Workers, electricians, contractors | Shock, fire, equipment failure | General authorization process. | 4 | 5 | 20 | Very High | Only authorized competent persons modify systems; maintain drawings/records; inspect and test changes before energization. | 1 | 5 | 5 | Medium | Electrical Engineer |
| 25 | Emergency electrical rescue | Delayed response after shock or electrical incident | Injured worker, rescuers | Secondary injury, fatality | First-aid arrangements. | 3 | 5 | 15 | High | Provide emergency procedure, competent first aid/rescue, safe isolation capability and communication; never touch casualty until electrical danger is controlled. | 1 | 5 | 5 | Medium | Site Manager / HSE |

Risk scores are calculated as Likelihood × Severity. Residual ratings assume the listed additional controls are implemented and verified.

# 12. Key Electrical Safety Requirements

| No. | Requirement | Expected Practice |
| --- | --- | --- |
| 1 | Planning | Identify circuits, equipment, voltage, energy sources, work method and interfaces before starting. |
| 2 | Competency | Use authorized and competent electrical personnel. |
| 3 | Isolation | De-energize, isolate, lock/tag and control all sources before work whenever practicable. |
| 4 | Test for Dead | Verify absence of voltage at the point of work using suitable test equipment. |
| 5 | Stored Energy | Identify and discharge capacitors, batteries, UPS and other stored-energy sources. |
| 6 | Protection | Maintain suitable enclosures, barriers, insulation, guarding and protective devices. |
| 7 | Earthing / Bonding | Verify protective earthing and bonding arrangements. |
| 8 | Temporary Power | Inspect and maintain temporary distribution, cables and protection devices. |
| 9 | Cables | Protect cables from water, vehicles, heat, sharp edges and mechanical damage. |
| 10 | Arc Flash | Assess arc-flash exposure and establish suitable boundaries/PPE where applicable. |
| 11 | Tools / Test Equipment | Use correctly rated, inspected and calibrated equipment. |
| 12 | Services | Identify overhead and underground electrical services before work. |
| 13 | Fire | Maintain electrical fire controls and clear access to isolation points. |
| 14 | Emergency | Maintain safe electrical rescue, first aid and emergency isolation arrangements. |
| 15 | Housekeeping | Keep electrical areas dry, clean, accessible and free of combustible materials. |
| 16 | Unauthorized Work | Prevent unapproved connections, modifications or energization. |

# 13. Inspection and Monitoring

Electrical installations, temporary power systems, tools and protective devices should be inspected and tested at the frequency required by applicable requirements, manufacturer instructions and site procedures. Defective equipment must be isolated, identified and removed from service until repaired or replaced by competent personnel.

| Inspection / Monitoring Activity | Minimum Focus |
| --- | --- |
| Distribution board inspection | Enclosure, covers, labeling, breakers, RCD/RCCB, earthing, connections and access. |
| Temporary power inspection | Cables, plugs, sockets, protection, routing, mechanical damage and water exposure. |
| Portable tool inspection | Cable, plug, casing, switch, guards and evidence of damage. |
| Isolation verification | Correct circuit/source, lock/tag, test-for-dead and stored-energy controls. |
| Test equipment inspection | Rating, condition, calibration/status, leads and probes. |
| Electrical work area inspection | Barriers, warning signs, housekeeping, fire controls and access. |
| Post-change verification | Testing and inspection after modification, repair or re-energization. |

# 14. Monitoring and Review of the Risk Assessment

| Review Trigger | Required Action |
| --- | --- |
| Before work starts | Confirm electrical system, work method, isolation and competent personnel. |
| Change in circuit / equipment | Update identification, isolation and risk controls. |
| New energy source identified | Stop affected work and revise isolation arrangements. |
| Temporary power change | Inspect and verify protection before use. |
| Wet / environmental change | Reassess suitability of equipment and protection. |
| Incident / near miss | Investigate and reassess relevant electrical hazards. |
| Design / modification change | Review drawings, protection, earthing and testing requirements. |
| Emergency event | Review response arrangements and lessons learned. |
| Periodic review | Confirm assessment remains suitable for the current work phase. |

# 15. Emergency and Rescue

Electrical emergencies require the electrical hazard to be controlled before rescuers approach the casualty. A person in contact with an energized source must not be touched until the source is isolated or otherwise made safe by a competent person. Emergency arrangements should cover isolation, first aid, rescue, fire response, communication and access for emergency responders.

| Emergency Element | Requirement |
| --- | --- |
| Electric shock | Do not touch casualty until electrical danger is controlled; isolate source and activate emergency response. |
| Arc flash | Isolate where safe, provide first aid for burns/injuries and control the area. |
| Electrical fire | Isolate power where safe, raise alarm and use appropriate fire response equipment. |
| Damaged service | Stop work, keep people clear and arrange competent service isolation. |
| Emergency communication | Maintain reliable communication and clear emergency access. |
| Post-incident | Provide medical/first-aid response, report the incident and review controls. |

# 16. Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Project / Construction Manager | Provide resources and ensure electrical safety controls are implemented. |
| Electrical Engineer / Supervisor | Plan and supervise electrical work, isolation and technical controls. |
| Authorized Person | Control electrical isolation, switching and authorization as applicable. |
| Electricians / Technicians | Follow approved procedures, use suitable equipment and report defects. |
| HSE Team | Monitor compliance, advise on controls and verify site arrangements. |
| Supervisors | Control access, housekeeping and work interfaces. |
| Workers | Do not interfere with electrical systems and report damaged equipment or unsafe conditions. |
| Contractors / Subcontractors | Comply with electrical requirements and provide task-specific assessments where required. |

# 17. Worker Briefing / Acknowledgement

Personnel involved in or affected by electrical work should be briefed on electrical hazards, isolation arrangements, access controls, PPE, temporary power controls, emergency response and stop-work requirements before starting.

| No. | Name | Position | Company | Signature | Date |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

# 18. Review and Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |

# 19. Disclaimer

This document is a general completed reference risk assessment for electrical work. It is not a substitute for project-specific electrical design, isolation procedures, permits, switching schedules, arc-flash studies, testing procedures, competent-person determinations or task-specific risk assessments. A competent electrical professional must review and adapt the assessment to the actual voltage, equipment, circuit configuration, energy sources, environment, work method, workforce competency, client requirements and applicable requirements. Energized work should only proceed when justified, authorized and appropriately controlled. Risk ratings must be verified against actual conditions and controls must be monitored throughout the work.