PROJECT NAME

EXCAVATION & TRENCHING
RISK ASSESSMENT

Comprehensive Risk Assessment for Excavation and Trenching Activities

| Document Information | Details |
| --- | --- |
| Resource Category | Risk Assessments |
| Document Type | Completed Practical Risk Assessment |
| Revision | Rev. 00 |
| Status | Reference Document |

Prepared as a practical HSE reference resource.

# 1. Document / Assessment Information

| Field | Details |
| --- | --- |
| Project / Site Name | ________________________________ |
| Location | ________________________________ |
| Client | ________________________________ |
| Main Contractor | ________________________________ |
| Subcontractor | ________________________________ |
| Assessment Reference | ________________________________ |
| Assessment Date | ________________________________ |
| Review Date | ________________________________ |
| Activity | Excavation and Trenching |
| Assessed By | ________________________________ |
| Position | ________________________________ |
| Reviewed By | ________________________________ |
| Approved By | ________________________________ |
| Persons at Risk | Workers, supervisors, operators, contractors, visitors and others who may be exposed |
| Revision | 00 |
| Document Status | Completed Reference Assessment |

# 2. Purpose

This assessment identifies foreseeable hazards associated with excavation and trenching activities, evaluates the associated risks, and establishes practical controls to prevent collapse, falls, struck-by incidents, underground service damage, equipment interactions, flooding, hazardous atmospheres and other excavation-related harm. It is a completed practical reference assessment and does not replace a project-specific assessment, approved excavation plan or competent-person evaluation.

# 3. Scope

The assessment covers excavation and trenching activities including planning and permit arrangements, utility identification, excavation setup, access and egress, temporary works and shoring, benching and battering, spoil placement, plant and vehicle interfaces, edge protection, underground services, water ingress, hazardous atmospheres, work near structures, lifting into excavations, night work, weather conditions, inspections and emergency response.

# 4. Persons at Risk

| No. | Person / Group | Potential Exposure |
| --- | --- | --- |
| 1 | Excavation workers | Collapse, falls, buried services, hazardous atmosphere and water ingress |
| 2 | Excavator / plant operators | Plant interaction, edge collapse and underground services |
| 3 | Banksmen / spotters | Plant movement, falling material and service damage |
| 4 | Supervisors / engineers / HSE personnel | Exposure during inspection and supervision |
| 5 | Workers near excavation edges | Falls, falling materials and plant interaction |
| 6 | Utility/service personnel | Damage to or contact with underground services |
| 7 | Delivery drivers / contractors | Vehicle and excavation interface |
| 8 | Visitors / unauthorized persons | Falls into open excavation |
| 9 | Emergency responders | Exposure during rescue or incident response |
| 10 | Nearby public / third parties | Exposure where excavations interface with public areas |

# 5. Excavation and Trenching Control Principles

Excavation work must be planned before breaking ground. The controls should address ground conditions, excavation geometry, nearby structures, underground services, water, access, plant, spoil, weather and emergency arrangements. Excavations should be designed and supported according to competent engineering assessment and applicable requirements. Workers must not enter an excavation where collapse or other hazards are not adequately controlled.

| Priority | Principle | Examples |
| --- | --- | --- |
| 1 | Avoid / reduce exposure | Avoid unnecessary excavation, minimize open duration and use alternative methods where reasonably practicable. |
| 2 | Prevent collapse and falls | Suitable battering/benching, shoring or engineered support; guardrails, barriers and controlled access. |
| 3 | Control interfaces | Locate services, segregate plant and people, control spoil and maintain exclusion zones. |
| 4 | Minimize consequences | Emergency rescue arrangements, safe access/egress, monitoring and prompt response to changing conditions. |

# 6. General Control Requirements

| No. | Control Area | General Requirement |
| --- | --- | --- |
| 1 | Planning | Plan the excavation sequence, dimensions, support, access, plant and emergency arrangements. |
| 2 | Competency | Use competent persons for excavation planning, inspections, service identification and specialist temporary works. |
| 3 | Permit / Authorization | Use required excavation permits and approvals before breaking ground. |
| 4 | Underground Services | Obtain current service information, locate and mark services, and use controlled excavation methods near them. |
| 5 | Ground Conditions | Assess soil/rock, groundwater, vibration, surcharge and nearby structures. |
| 6 | Protective Systems | Provide engineered shoring, shielding, benching or battering as required. |
| 7 | Access / Egress | Provide safe ladders, stairs or ramps at suitable locations. |
| 8 | Edge Protection | Prevent falls using barriers, guardrails and controlled access. |
| 9 | Spoil / Materials | Keep spoil, materials and equipment away from excavation edges as determined by competent assessment. |
| 10 | Plant Interface | Keep mobile plant away from unsupported edges and use banksmen where required. |
| 11 | Water | Control groundwater, rainwater and flooding before and during entry. |
| 12 | Atmosphere | Assess oxygen deficiency, toxic or flammable atmospheres where reasonably foreseeable. |
| 13 | Inspection | Inspect excavations before entry, after changes and after events affecting stability. |
| 14 | Weather | Reassess after heavy rain, flooding, vibration or other significant changes. |
| 15 | Emergency | Maintain rescue, communication and emergency response arrangements. |

# 7. Equipment and Systems Covered

The assessment considers excavation plant and protective systems such as excavators, loaders, dumpers, trench boxes, shoring systems, hydraulic supports, sheet piles, benching and battering arrangements, ladders, stairs, ramps, barriers, gas detectors, pumps, temporary lighting and service-detection equipment. Equipment and temporary works must be suitable for the actual ground conditions, excavation geometry and work method.

# 8. Risk Assessment Methodology

Risk is assessed using a 5 × 5 matrix. The risk score is calculated as Likelihood × Severity. Initial risk represents the exposure before additional controls are applied; residual risk represents the expected risk after the specified controls are implemented and verified.

| Likelihood | Description |
| --- | --- |
| 1 | Rare |
| 2 | Unlikely |
| 3 | Possible |
| 4 | Likely |
| 5 | Almost Certain |

| Severity | Description |
| --- | --- |
| 1 | Minor injury |
| 2 | First-aid injury |
| 3 | Medical treatment injury |
| 4 | Serious / major injury |
| 5 | Fatality / catastrophic consequence |

# 9. Risk Classification

| Score | Risk Level | General Action |
| --- | --- | --- |
| 1–4 | Low | Maintain controls and monitor. |
| 5–9 | Medium | Improve controls where reasonably practicable. |
| 10–16 | High | Immediate management attention and additional controls required. |
| 17–25 | Very High | Work should not proceed until adequate controls reduce the risk. |

# 10. 5 × 5 Risk Matrix

| Likelihood ↓ / Severity → | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 Almost Certain | 5 | 10 | 15 | 20 | 25 |
| 4 Likely | 4 | 8 | 12 | 16 | 20 |
| 3 Possible | 3 | 6 | 9 | 12 | 15 |
| 2 Unlikely | 2 | 4 | 6 | 8 | 10 |
| 1 Rare | 1 | 2 | 3 | 4 | 5 |

# 11. Detailed Excavation & Trenching Risk Assessment

The following assessment addresses representative excavation and trenching hazards. Risk ratings are reference values and must be verified against actual ground conditions, excavation geometry, protective systems, equipment, services and work methods before implementation.

| No. | Activity / Hazard | Hazard Description | Persons at Risk | Potential Consequences | Existing Control Measures | Initial L | Initial S | Initial Score | Initial Level | Further Controls | Residual L | Residual S | Residual Score | Residual Level | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Excavation collapse | Unsupported or inadequately supported excavation sidewalls | Excavation workers, supervisors, rescuers | Burial, crushing, serious injury or fatality | Competent planning; basic ground assessment; access control. | 4 | 5 | 20 | Very High | Provide engineered shoring, shielding, benching or battering based on competent assessment; inspect before entry and after changes. | 1 | 5 | 5 | Medium | Competent Person / Construction Manager |
| 2 | Trench collapse | Workers enter trench with inadequate protective system | Workers, supervisors | Burial or crushing | Trench protection and supervision where provided. | 4 | 5 | 20 | Very High | Do not permit entry until protective system is installed and verified; maintain safe dimensions and access; prohibit unsupported entry. | 1 | 5 | 5 | Medium | Excavation Supervisor |
| 3 | Underground electrical services | Excavation contacts buried electrical cable | Workers, operators, utility personnel | Electrocution, arc flash, fire, outage | Service drawings and detection where available. | 4 | 5 | 20 | Very High | Obtain current utility information; scan and mark services; use controlled hand excavation near identified services; establish exclusion distances and emergency procedure. | 1 | 5 | 5 | Medium | Permit Holder / Competent Person |
| 4 | Gas / fuel pipeline | Excavation damages buried gas or fuel line | Workers, public, utility personnel | Fire, explosion, toxic exposure, fatality | Utility information and service locating. | 3 | 5 | 15 | High | Confirm service location and depth; controlled excavation; no ignition sources where required; emergency isolation/contact arrangements. | 1 | 5 | 5 | Medium | Permit Holder / Supervisor |
| 5 | Water / pressurized service | Damage to buried water or pressurized line | Workers, nearby personnel | Flooding, engulfment, injury, service disruption | Service records and detection. | 3 | 4 | 12 | High | Verify route; expose service carefully; maintain controlled excavation and emergency isolation plan. | 1 | 4 | 4 | Low | Supervisor |
| 6 | Unknown services | Unidentified service encountered during excavation | Workers, operators | Service strike, injury, outage or fire | Stop-work instruction for unexpected obstructions. | 4 | 5 | 20 | Very High | Stop excavation; isolate area; identify service through competent utility process; update permit and controls before resuming. | 1 | 5 | 5 | Medium | Permit Holder |
| 7 | Falls into excavation | Open excavation without adequate barrier or cover | Workers, visitors, drivers | Fall to lower level, serious injury or fatality | Barricades and warning signs where provided. | 4 | 5 | 20 | Very High | Install robust guardrails/barriers; cover openings where appropriate; control access; provide lighting and signage. | 1 | 5 | 5 | Medium | Supervisor / HSE |
| 8 | Plant near excavation edge | Excavator, dumper or vehicle operates too close to edge | Operators, workers | Edge collapse, overturning, struck-by incident | Basic exclusion zones and operator awareness. | 4 | 5 | 20 | Very High | Establish engineered stand-off distances; use physical barriers/stop blocks where required; banksman; assess ground loading and surcharge. | 1 | 5 | 5 | Medium | Plant Supervisor |
| 9 | Spoil heap at edge | Spoil or materials stored too close to excavation | Workers, plant operators | Edge surcharge, collapse, falling material | Housekeeping and supervisor monitoring. | 4 | 5 | 20 | Very High | Set competent-person-defined setback; keep heavy materials away from edge; remove excess spoil promptly. | 1 | 5 | 5 | Medium | Excavation Supervisor |
| 10 | Water ingress / flooding | Groundwater, rainwater or burst service enters excavation | Workers, plant operators | Engulfment, collapse, drowning, electrical hazards | Pumps and weather monitoring where applicable. | 4 | 5 | 20 | Very High | Provide drainage/dewatering plan; inspect after rain; maintain pumps and backup arrangements; prohibit entry when unsafe. | 1 | 5 | 5 | Medium | Construction Manager |
| 11 | Hazardous atmosphere | Oxygen deficiency, toxic or flammable gas in excavation | Workers, rescuers | Asphyxiation, poisoning, fire or explosion | Atmospheric assessment where required. | 3 | 5 | 15 | High | Conduct atmospheric testing where foreseeable; provide ventilation; use calibrated detector; establish entry/evacuation criteria and rescue plan. | 1 | 5 | 5 | Medium | Competent Person / HSE |
| 12 | Poor access and egress | Inadequate or blocked ladder/stair access | Workers | Falls, delayed evacuation | Ladders or access points provided. | 3 | 4 | 12 | High | Provide secure, suitable access at appropriate intervals; keep routes clear; inspect and maintain access. | 1 | 4 | 4 | Low | Supervisor |
| 13 | Inadequate shoring / support | Incorrect installation, damaged or unsuitable temporary works | Workers, supervisors | Collapse and burial | Temporary support system provided. | 4 | 5 | 20 | Very High | Use engineered/design-approved system; competent installation; inspect components; prohibit unauthorized modification; maintain sequence. | 1 | 5 | 5 | Medium | Temporary Works Coordinator / Supervisor |
| 14 | Benching / battering failure | Incorrect slope or bench geometry for ground conditions | Workers | Sidewall collapse | Basic excavation profile controls. | 4 | 5 | 20 | Very High | Use competent engineering assessment; maintain specified angles/bench dimensions; inspect after changes and weather events. | 1 | 5 | 5 | Medium | Competent Person |
| 15 | Adjacent structure instability | Excavation undermines building, wall, road or utility support | Workers, public, adjacent occupants | Structural collapse, burial, major damage | Pre-work survey and visual monitoring. | 3 | 5 | 15 | High | Assess influence zone; provide engineered support/underpinning where required; monitor movement and establish exclusion zones. | 1 | 5 | 5 | Medium | Engineer / Construction Manager |
| 16 | Vibration from plant | Vibration affects excavation stability or adjacent structures | Workers, operators, public | Collapse, structural damage | Plant controls and supervision. | 3 | 4 | 12 | High | Assess vibration-sensitive conditions; control plant location and operating methods; monitor stability and structures where required. | 1 | 4 | 4 | Low | Construction Manager |
| 17 | Falling materials into excavation | Tools, materials or debris fall from edge | Workers in excavation | Struck-by injury, serious injury | Housekeeping and basic barriers. | 4 | 4 | 16 | High | Use edge barriers/toe boards where appropriate; secure materials; maintain exclusion zones; prohibit throwing materials. | 1 | 4 | 4 | Low | Supervisor |
| 18 | Lifting into / out of excavation | Loads suspended over workers or excavation | Workers, lifting team | Dropped load, crushing, equipment interaction | Lifting equipment and operator controls. | 3 | 5 | 15 | High | Plan lift; prevent people under suspended loads; use competent lifting team; maintain exclusion zone and communication. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 19 | Excavator swing radius | Workers enter plant operating/swing area | Workers, banksmen, operators | Struck-by/crushing fatality | Banksman and operator communication. | 4 | 5 | 20 | Very High | Establish exclusion zone; segregate pedestrians; use dedicated banksman; maintain visibility and communication; no unauthorized entry. | 1 | 5 | 5 | Medium | Plant Supervisor |
| 20 | Blind spots / poor visibility | Operator cannot see workers around excavation | Workers, operators | Struck-by/crushing | Mirrors/cameras and banksman where used. | 4 | 5 | 20 | Very High | Improve visibility; use competent banksman; establish communication signals; stop plant when personnel enter controlled area. | 1 | 5 | 5 | Medium | Plant Supervisor |
| 21 | Night / low-light excavation | Insufficient lighting around excavation | Workers, operators, drivers | Falls, collisions, plant incidents | Temporary lighting where provided. | 3 | 4 | 12 | High | Provide adequate task and perimeter lighting; illuminate access, barriers and work zones; inspect lighting before shift. | 1 | 4 | 4 | Low | Site Supervisor |
| 22 | Adverse weather | Heavy rain, wind, heat or poor visibility changes excavation conditions | Workers, operators | Collapse, flooding, slips, heat illness | Weather monitoring and site inspections. | 4 | 5 | 20 | Very High | Stop and reassess when conditions change; inspect after rain; maintain drainage; provide heat controls and safe restart criteria. | 2 | 5 | 10 | High | Site Manager / HSE |
| 23 | Unauthorized access | Workers or public enter excavation without authorization | Workers, visitors, public | Fall, collapse exposure | Barriers and signs. | 3 | 5 | 15 | High | Use robust barriers, controlled access points, signage, security checks and maintain protection outside working hours. | 1 | 5 | 5 | Medium | Supervisor / Security |
| 24 | Emergency rescue | Buried worker, collapse, flooding or service strike requires rescue | Workers, rescuers | Fatality, secondary collapse, electrical exposure | Emergency contacts and general site response. | 4 | 5 | 20 | Very High | Prepare excavation emergency plan; do not enter unstable excavation for improvised rescue; provide competent rescue arrangements, communication and emergency access. | 1 | 5 | 5 | Medium | Site Manager / HSE |
| 25 | Excavation inspection failure | Changes or deterioration not identified before entry | Workers, supervisors | Exposure to collapse, water or other hazards | Routine inspections. | 4 | 5 | 20 | Very High | Competent person to inspect before entry and after rain, vibration, alteration or other events; record findings and close unsafe areas. | 1 | 5 | 5 | Medium | Competent Person |

Risk scores are calculated as Likelihood × Severity. Residual ratings assume all listed additional controls are implemented and verified.

# 12. Key Excavation & Trenching Safety Requirements

| No. | Requirement | Expected Practice |
| --- | --- | --- |
| 1 | Planning | Plan excavation sequence, support, services, access, plant and emergency response before starting. |
| 2 | Permit / Authorization | Complete required excavation permit and approvals before breaking ground. |
| 3 | Competency | Use competent personnel for excavation inspections, utility locating and temporary works. |
| 4 | Ground Assessment | Assess soil, rock, groundwater, surcharge, vibration and adjacent structures. |
| 5 | Protective Systems | Provide suitable shoring, shielding, benching or battering based on competent assessment. |
| 6 | Services | Locate, mark and safely expose underground services using approved methods. |
| 7 | Access / Egress | Provide secure ladders, stairs or ramps and keep them clear. |
| 8 | Edge Protection | Prevent falls with robust barriers and controlled access. |
| 9 | Spoil / Surcharge | Keep spoil, materials and plant at safe distances from edges. |
| 10 | Plant Interface | Segregate people and plant; control edge loading and use banksmen. |
| 11 | Water | Control groundwater, rainwater and flooding before entry. |
| 12 | Atmosphere | Test and control hazardous atmospheres where reasonably foreseeable. |
| 13 | Inspection | Inspect before entry and after changes, rain, vibration or other stability-affecting events. |
| 14 | Falling Objects | Control tools, materials and debris around excavation edges. |
| 15 | Emergency | Maintain practical rescue, communication and emergency access arrangements. |
| 16 | Weather | Stop and reassess work when weather changes excavation stability or visibility. |

# 13. Inspection and Monitoring

Excavations and protective systems should be inspected by a competent person before workers enter and whenever conditions change. Reinspection should occur after rain, flooding, vibration, nearby construction activity, alteration of support systems, material changes or other events that could affect stability. Defects or unsafe conditions must be controlled immediately and the excavation kept out of service until safe.

| Inspection / Monitoring Activity | Minimum Focus |
| --- | --- |
| Pre-entry excavation inspection | Ground condition, sidewalls, support, edge protection, access and water. |
| Protective system inspection | Shoring, trench box, hydraulic supports, benching/battering and signs of movement. |
| Underground service verification | Service markings, exposed services, protection and permit conditions. |
| Plant and edge monitoring | Stand-off distance, surcharge, exclusion zone, ground stability and banksman controls. |
| Atmospheric monitoring | Oxygen, toxic/flammable gases and ventilation where required. |
| Weather / event inspection | Reassess after rain, flooding, vibration or other significant changes. |
| HSE / supervisor inspection | Housekeeping, barriers, access, signage, unauthorized entry and changing hazards. |

# 14. Monitoring and Review of the Risk Assessment

| Review Trigger | Required Action |
| --- | --- |
| Before work starts | Confirm current services, ground conditions, support system, access and emergency arrangements. |
| Change in excavation depth or geometry | Review stability, support and access before continuing. |
| New service identified | Stop affected work, protect the area and revise controls. |
| Change in soil / groundwater | Reassess excavation stability and dewatering requirements. |
| Heavy rain / flooding | Stop entry as necessary and reinspect before restart. |
| Nearby construction / vibration | Reassess stability and adjacent structures. |
| Plant or loading change | Review surcharge and edge stand-off distances. |
| Incident / near miss | Investigate and reassess relevant hazards and controls. |
| Periodic review | Confirm the assessment remains suitable for the current work phase. |

# 15. Emergency and Rescue

Excavation emergencies can involve collapse, burial, flooding, hazardous atmospheres or underground service strikes. Emergency arrangements must be established before work begins and must prevent rescuers from becoming additional casualties. Personnel must not enter an unstable excavation for an improvised rescue. The response should include communication, isolation of services where necessary, competent rescue arrangements, emergency access and first aid.

| Emergency Element | Requirement |
| --- | --- |
| Collapse | Stop work, isolate the area and keep personnel away from unstable ground; activate competent rescue response. |
| Service strike | Stop work, keep people clear, avoid contact with damaged services and contact the responsible utility/emergency response. |
| Flooding | Evacuate, isolate water source where safe, control pumps and prevent re-entry until inspected. |
| Hazardous atmosphere | Evacuate, prevent ignition where relevant and use trained responders with suitable equipment. |
| Communication | Maintain reliable emergency communication and clear site access for responders. |
| Post-incident | Provide first aid, report the incident, preserve evidence where appropriate and review controls. |

# 16. Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Project / Construction Manager | Provide resources and ensure excavation controls are implemented. |
| HSE Manager / HSE Team | Review, monitor and verify controls; support inspections and emergency planning. |
| Excavation Supervisor | Control work sequence, access, protective systems and daily site conditions. |
| Competent Person | Inspect excavation, assess changing conditions and determine necessary controls. |
| Temporary Works Coordinator / Engineer | Coordinate engineered shoring, support or adjacent-structure protection where required. |
| Plant Supervisor / Operator | Operate equipment safely, maintain exclusion zones and follow banksman instructions. |
| Banksman / Spotter | Control plant movements and protect personnel in the plant interface zone. |
| Workers | Follow procedures, stay within controlled areas and report hazards or changes. |
| Contractors / Subcontractors | Comply with excavation requirements and provide task-specific assessments where required. |

# 17. Worker Briefing / Acknowledgement

Personnel affected by excavation work should be briefed on the excavation plan, access and egress, protective systems, service hazards, exclusion zones, plant interfaces, emergency arrangements and stop-work requirements before starting.

| No. | Name | Position | Company | Signature | Date |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

# 18. Review and Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |

# 19. Disclaimer

This document is a general completed reference risk assessment for excavation and trenching. It is not a substitute for a project-specific excavation plan, permit, engineering design, temporary works assessment, utility survey or task-specific risk assessment. A competent person must review and adapt the assessment to actual ground conditions, excavation dimensions, protective systems, underground services, equipment, workforce competency, environmental conditions, client requirements and applicable requirements. Additional specialist assessments may be required. Risk ratings must be verified against actual conditions and controls must be monitored throughout the work.