PROJECT NAME

LIFTING OPERATIONS
RISK ASSESSMENT

Comprehensive Risk Assessment for Lifting and Material Handling Activities

| Document Information | Details |
| --- | --- |
| Resource Category | Risk Assessments |
| Document Type | Completed Practical Risk Assessment |
| Revision | Rev. 00 |
| Status | Reference Document |

Prepared as a practical HSE reference resource.

# 1. Document / Assessment Information

| Field | Details |
| --- | --- |
| Project / Site Name |  |
| Location |  |
| Client |  |
| Main Contractor |  |
| Subcontractor |  |
| Assessment Reference |  |
| Assessment Date |  |
| Review Date |  |
| Activity | Lifting Operations |
| Assessed By |  |
| Position |  |
| Reviewed By |  |
| Approved By |  |
| Persons at Risk | Crane operators, riggers, signalers, workers, supervisors, drivers, contractors, visitors and others who may be exposed |
| Revision | 00 |
| Document Status | Completed Reference Assessment |

# 2. Purpose

This assessment identifies foreseeable hazards associated with lifting operations and material handling, evaluates the potential risks, and establishes practical controls for safe planning, equipment selection, lifting accessory use, load handling, communication, exclusion zones and emergency response. It is a completed practical reference assessment and does not replace a project-specific lifting plan, lift study, equipment inspection, manufacturer requirements or competent-person assessment.

# 3. Scope

The assessment covers mobile cranes, crawler and tower cranes, telehandlers where used for lifting, lifting accessories, rigging and slinging, load assessment, lifting plans, ground conditions, crane setup, outrigger deployment, overhead services, suspended loads, personnel exclusion, communication, tandem lifts, blind lifts, weather, night lifting, delivery interfaces, storage and emergency arrangements.

# 4. Persons at Risk

| No. | Person / Group | Potential Exposure |
| --- | --- | --- |
| 1 | Crane operators | Equipment instability, overload, collision and suspended-load incidents |
| 2 | Riggers / slingers | Dropped loads, pinch points and incorrect rigging |
| 3 | Banksmen / signalers | Struck-by incidents and communication failures |
| 4 | Workers near lifting area | Dropped or swinging loads and crane movement |
| 5 | Supervisors / lifting team | Exposure during setup, inspection and execution |
| 6 | Drivers / delivery personnel | Vehicle and lifting interface |
| 7 | Maintenance personnel | Unexpected movement, stored energy and equipment hazards |
| 8 | Visitors / unauthorized persons | Entry into lifting exclusion zones |
| 9 | Public / third parties | Exposure where lifting occurs near public interfaces |
| 10 | Emergency responders | Exposure during recovery of failed loads or equipment |

# 5. Lifting Control Principles

Lifting operations must be planned and executed by competent personnel using suitable, inspected equipment and lifting accessories. The lift should be assessed for load weight, dimensions, center of gravity, radius, height, travel path, ground conditions, environmental conditions and nearby hazards. People must be protected from suspended-load and equipment hazards through exclusion, controlled access and clear communication.

| Priority | Principle | Examples |
| --- | --- | --- |
| 1 | Avoid unnecessary lifting | Use ground-level assembly or mechanical handling alternatives where reasonably practicable. |
| 2 | Plan and control | Use competent lifting plans, suitable equipment, verified load data and defined roles. |
| 3 | Prevent dropped-load events | Inspect accessories, secure loads, use correct rigging and control movement. |
| 4 | Protect people | Establish exclusion zones, control access and prohibit people under suspended loads. |
| 5 | Minimize consequences | Emergency stop, recovery arrangements and contingency plans for abnormal conditions. |

# 6. General Control Requirements

| No. | Control Area | General Requirement |
| --- | --- | --- |
| 1 | Lift Planning | Prepare a suitable lift plan appropriate to complexity and risk. |
| 2 | Competency | Use competent operators, riggers, signalers and lifting supervisors. |
| 3 | Equipment Selection | Select equipment with adequate rated capacity and suitable configuration. |
| 4 | Load Information | Confirm weight, dimensions, center of gravity and lifting points. |
| 5 | Accessories | Use suitable, inspected and compatible slings, shackles, hooks and accessories. |
| 6 | Ground Conditions | Verify bearing capacity, level, stability and suitability for crane setup. |
| 7 | Crane Setup | Correctly position crane, outriggers, mats and counterweights as required. |
| 8 | Exclusion Zone | Establish and maintain a controlled lifting area. |
| 9 | Communication | Use agreed signals/radio communication and a designated signaler. |
| 10 | Overhead Hazards | Identify power lines, structures, obstructions and other interfaces. |
| 11 | Weather | Monitor wind, rain, visibility and manufacturer operating limits. |
| 12 | Trial Lift | Conduct controlled trial lift where required to verify balance and rigging. |
| 13 | Suspended Loads | Never permit personnel beneath suspended loads. |
| 14 | Inspection | Conduct pre-use and required periodic inspections. |
| 15 | Emergency | Maintain stop-work and emergency response arrangements. |

# 7. Equipment and Systems Covered

The assessment considers cranes, lifting beams, spreader beams, hoists, winches, telehandlers used within their approved lifting function, forklifts where permitted for lifting, slings, shackles, hooks, lifting points, chains, wire ropes, webbing slings, tag lines, outriggers, mats, radios and other lifting accessories. Equipment must be used within manufacturer and certification requirements.

# 8. Risk Assessment Methodology

Risk is assessed using a 5 × 5 matrix. Risk score = Likelihood × Severity. Initial risk represents exposure before additional controls; residual risk represents the expected risk after the listed additional controls are implemented and verified.

| Likelihood | Description |
| --- | --- |
| 1 | Rare |
| 2 | Unlikely |
| 3 | Possible |
| 4 | Likely |
| 5 | Almost Certain |

| Severity | Description |
| --- | --- |
| 1 | Minor injury |
| 2 | First-aid injury |
| 3 | Medical treatment injury |
| 4 | Serious / major injury |
| 5 | Fatality / catastrophic consequence |

# 9. Risk Classification

| Score | Risk Level | General Action |
| --- | --- | --- |
| 1–4 | Low | Maintain controls and monitor. |
| 5–9 | Medium | Improve controls where reasonably practicable. |
| 10–16 | High | Immediate management attention and additional controls required. |
| 17–25 | Very High | Work should not proceed until adequate controls reduce the risk. |

# 10. 5 × 5 Risk Matrix

| Likelihood ↓ / Severity → | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 Almost Certain | 5 | 10 | 15 | 20 | 25 |
| 4 Likely | 4 | 8 | 12 | 16 | 20 |
| 3 Possible | 3 | 6 | 9 | 12 | 15 |
| 2 Unlikely | 2 | 4 | 6 | 8 | 10 |
| 1 Rare | 1 | 2 | 3 | 4 | 5 |

# 11. Detailed Lifting Operations Risk Assessment

The following assessment addresses representative lifting-operation hazards. Risk ratings are reference values and must be verified against the actual lift plan, load, equipment, ground conditions, environment and work area before implementation.

| No. | Activity / Hazard | Hazard Description | Persons at Risk | Potential Consequences | Existing Control Measures | Initial L | Initial S | Initial Score | Initial Level | Further Controls | Residual L | Residual S | Residual Score | Residual Level | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Unplanned lifting operation | Lift begins without adequate planning or authorization | Lifting team, workers, third parties | Dropped load, collision, equipment failure, fatality | General lifting procedure and supervision. | 4 | 5 | 20 | Very High | Use approved lift plan appropriate to complexity; confirm roles, load data, equipment, route, ground and exclusion zone before lifting. | 1 | 5 | 5 | Medium | Lifting Supervisor / Construction Manager |
| 2 | Incorrect load weight | Actual load exceeds crane/accessory capacity | Operator, riggers, workers | Overload, structural failure, dropped load, fatality | Load information available where known. | 3 | 5 | 15 | High | Verify load weight and center of gravity using reliable data; include rigging weight; do not lift when capacity is uncertain. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 3 | Incorrect load center of gravity | Load tilts or becomes unstable during lift | Riggers, workers | Dropped or swinging load | Basic rigging and trial lift. | 3 | 5 | 15 | High | Determine center of gravity; select suitable lifting points; use controlled trial lift; adjust rigging before proceeding. | 1 | 5 | 5 | Medium | Rigger / Lifting Supervisor |
| 4 | Crane overload | Lift exceeds rated capacity for actual radius/configuration | Operator, lifting team | Crane failure, overturning, fatality | Load chart available to operator. | 4 | 5 | 20 | Very High | Verify load chart, radius, boom configuration and capacity; account for accessories and dynamic effects; use load moment protection as applicable. | 1 | 5 | 5 | Medium | Crane Operator / Lifting Supervisor |
| 5 | Poor ground conditions | Insufficient bearing capacity or unstable setup area | Crane operator, workers | Crane settlement, overturning, load drop | Ground visually checked. | 4 | 5 | 20 | Very High | Competent ground assessment; verify bearing capacity; use suitable mats/cribbing; maintain level setup and monitor settlement. | 1 | 5 | 5 | Medium | Lift Planner / Engineer |
| 6 | Outrigger failure / setup | Outriggers not fully deployed, supported or level | Operator, workers | Crane overturning, fatality | Outriggers used as required. | 4 | 5 | 20 | Very High | Deploy per manufacturer/lift plan; use rated mats; verify level and full support; establish exclusion around outriggers. | 1 | 5 | 5 | Medium | Crane Supervisor |
| 7 | Defective lifting accessories | Slings, shackles, hooks or chains damaged/expired | Riggers, workers | Accessory failure and dropped load | Pre-use visual checks. | 4 | 5 | 20 | Very High | Inspect before use; verify certification/traceability where required; remove damaged accessories from service; use correct WLL and configuration. | 1 | 5 | 5 | Medium | Lifting Supervisor / Rigger |
| 8 | Incorrect sling configuration | Sling angle or hitch reduces capacity or damages sling | Riggers, workers | Sling failure, dropped load | Basic rigging practice. | 4 | 5 | 20 | Very High | Select correct hitch and WLL; account for sling angle; protect from sharp edges; use compatible shackles and lifting points. | 1 | 5 | 5 | Medium | Competent Rigger |
| 9 | Hook / latch issue | Hook damaged, unlatched or side-loaded | Riggers, workers | Load disengagement | Hook and latch checked. | 3 | 5 | 15 | High | Inspect hook and latch; ensure correct seating; prohibit side loading; use appropriate hook configuration. | 1 | 5 | 5 | Medium | Rigger |
| 10 | Suspended load over people | Personnel enter area beneath suspended load | Workers, visitors | Crushing or fatality | General warning and supervision. | 4 | 5 | 20 | Very High | Establish physical exclusion zone; prohibit access beneath loads; use barriers, signage and dedicated spotters. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 11 | Swinging / uncontrolled load | Wind, rotation or poor control causes load to swing | Workers, structures, plant | Struck-by injury or collision | Tag lines where used. | 4 | 4 | 16 | High | Use suitable tag lines where safe; control speed; maintain clear route; account for wind and load shape; keep personnel clear. | 1 | 4 | 4 | Low | Lifting Team |
| 12 | Blind lift | Operator cannot directly see load or landing area | Operator, riggers, workers | Collision, dropped load, trapping | Radio communication where available. | 4 | 5 | 20 | Very High | Use designated signaler(s), tested radio/hand signals and controlled movement; stop on loss of communication. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 13 | Poor communication | Conflicting signals or radio failure | Operator, riggers, workers | Unexpected movement, collision, dropped load | Standard hand signals. | 4 | 5 | 20 | Very High | Designate one signaler; brief signals; test radios; stop lift if communication is lost or unclear. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 14 | Overhead power lines | Crane/load approaches energized electrical lines | Operator, riggers, workers, public | Electrocution, fire, fatality | Power line awareness. | 4 | 5 | 20 | Very High | Identify lines before lift; maintain required clearance; use approved isolation/de-energization where possible; establish controlled zones and dedicated observer. | 1 | 5 | 5 | Medium | Site Manager / HSE |
| 15 | Obstructions / structures | Load or boom contacts buildings, scaffolds or structures | Workers, public, property | Collision, falling materials, structural damage | Route checked before lifting. | 3 | 5 | 15 | High | Plan travel/slew path; remove obstructions where possible; use spotters; maintain clearance and controlled movement. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 16 | Weather / high wind | Wind affects crane stability or load control | Operator, lifting team | Load swing, overturning, dropped load | Weather monitored. | 4 | 5 | 20 | Very High | Follow manufacturer wind limits and lift plan; monitor gusts; suspend lift when limits/visibility are exceeded; secure load during stoppage. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 17 | Night lifting / poor visibility | Insufficient lighting or visibility | Operator, riggers, workers | Collision, wrong landing, dropped load | Temporary lighting available. | 3 | 4 | 12 | High | Provide adequate task and route lighting; illuminate signals and landing area; stop if visibility becomes inadequate. | 1 | 4 | 4 | Low | Site Supervisor |
| 18 | Tandem / multiple crane lift | Uneven load sharing or synchronized movement failure | Operators, riggers, workers | Overload, load drop, crane instability | Specialist supervision where used. | 4 | 5 | 20 | Very High | Use engineered lift plan with load-share calculations; appoint competent lift director; synchronized communication and controlled movement. | 1 | 5 | 5 | Medium | Lift Director / Engineer |
| 19 | Lifting personnel | Personnel lifted without suitable approved system | Workers, operators | Fall, crushing, fatality | General lifting equipment available. | 3 | 5 | 15 | High | Prohibit personnel lifting unless specifically permitted and using an approved personnel-lifting system with dedicated assessment and controls. | 1 | 5 | 5 | Medium | Construction Manager |
| 20 | Pinch / crush points | Hands/body caught between load and landing area | Riggers, workers | Crush injury, amputation | Gloves and supervision. | 4 | 4 | 16 | High | Use tag lines/handling aids; keep hands out of pinch points; establish landing zone and controlled positioning method. | 1 | 4 | 4 | Low | Rigger / Supervisor |
| 21 | Load not secured | Loose components or materials fall from lifted load | Workers below | Struck-by injury or fatality | General securing practices. | 4 | 5 | 20 | Very High | Secure all loose items; use suitable containers/baskets; verify load stability before lift; establish exclusion zone. | 1 | 5 | 5 | Medium | Rigger |
| 22 | Crane collision / interface | Two cranes or mobile plant operate in overlapping zones | Operators, workers | Collision, dropped load, fatality | Site coordination. | 3 | 5 | 15 | High | Coordinate lift schedules and zones; use anti-collision controls where applicable; dedicated communication and exclusion areas. | 1 | 5 | 5 | Medium | Lifting Manager |
| 23 | Transport / delivery interface | Vehicles enter lifting zone during operation | Drivers, lifting team | Collision or struck-by incident | Traffic management controls. | 4 | 4 | 16 | High | Segregate traffic; close access during critical lift; use banksman and barriers; coordinate delivery timing. | 1 | 4 | 4 | Low | Logistics Supervisor |
| 24 | Stored / landed load instability | Load becomes unstable after placement | Workers, riggers | Load collapse, crush injury | Landing area checked. | 3 | 5 | 15 | High | Verify stable foundation/support; use dunnage/chocks as required; release rigging only after load is stable. | 1 | 5 | 5 | Medium | Lifting Supervisor |
| 25 | Emergency / equipment failure | Crane or lifting accessory fails during operation | Operator, workers, public | Dropped load, entrapment, fatality | Emergency stop and site response arrangements. | 4 | 5 | 20 | Very High | Establish emergency response and exclusion arrangements; stop movement; isolate area; use competent recovery plan; never improvise under suspended loads. | 1 | 5 | 5 | Medium | Site Manager / Lifting Supervisor |

Risk scores are calculated as Likelihood × Severity. Residual ratings assume the listed additional controls are implemented and verified.

# 12. Key Lifting Safety Requirements

| No. | Requirement | Expected Practice |
| --- | --- | --- |
| 1 | Lift Planning | Prepare an appropriate lift plan and confirm load, equipment, route, ground and environmental conditions. |
| 2 | Competency | Use competent lifting supervisors, operators, riggers and signalers. |
| 3 | Equipment Capacity | Verify rated capacity for the actual configuration, radius and conditions. |
| 4 | Load Information | Confirm weight, dimensions, center of gravity and lifting points. |
| 5 | Accessories | Use suitable, compatible, inspected and correctly rated lifting accessories. |
| 6 | Ground Conditions | Verify crane setup area and use mats/cribbing as required. |
| 7 | Communication | Use one designated signaler and agreed communication method. |
| 8 | Exclusion Zone | Prevent unauthorized entry and keep people away from suspended loads. |
| 9 | Overhead Hazards | Identify power lines, structures and other obstructions before lifting. |
| 10 | Weather | Monitor wind, visibility and manufacturer limits. |
| 11 | Load Control | Use suitable tag lines and controlled movement where appropriate. |
| 12 | Landing | Ensure landing area is stable and personnel use safe positioning methods. |
| 13 | Inspection | Inspect cranes and accessories before use and at required intervals. |
| 14 | Emergency | Maintain stop-work and recovery arrangements for abnormal situations. |
| 15 | No People Under Loads | Never permit personnel under suspended loads. |
| 16 | Coordination | Coordinate simultaneous lifting, traffic and nearby activities. |

# 13. Inspection and Monitoring

Lifting equipment and accessories should be inspected before use and at the frequency required by applicable requirements, manufacturer instructions and site procedures. Defective equipment must be immediately removed from service, identified and controlled. The lifting supervisor should continuously monitor the lift for changing conditions, communication, exclusion zones, load stability and equipment behavior.

| Inspection / Monitoring Activity | Minimum Focus |
| --- | --- |
| Crane pre-use inspection | Structural condition, controls, alarms, hooks, wire ropes, brakes, outriggers and safety devices. |
| Lifting accessory inspection | Slings, shackles, hooks, chains, lifting beams, tags/identification and visible damage. |
| Ground / setup inspection | Level, bearing capacity, mats, outriggers, clearances and stability. |
| Lift area inspection | Exclusion zone, route, landing area, overhead hazards and traffic interfaces. |
| Communication check | Radio function, agreed signals and designated signaler. |
| Weather monitoring | Wind, visibility, rain and manufacturer/site limits. |
| Post-change verification | Reassess after relocation, equipment change, weather change or abnormal event. |

# 14. Monitoring and Review of the Risk Assessment

| Review Trigger | Required Action |
| --- | --- |
| Before each lifting operation | Confirm lift plan, equipment, load, route, ground and personnel. |
| Change in load | Recalculate capacity and revise rigging/lift plan where necessary. |
| Change in crane configuration/radius | Reconfirm load chart and lifting capacity. |
| Change in lifting area | Reassess ground, obstructions, services and exclusion zones. |
| Weather deterioration | Stop or reassess lift against operating limits. |
| New obstruction / interface | Stop affected movement and revise route/controls. |
| Equipment defect | Stop lift, isolate equipment and replace/repair before use. |
| Incident / near miss | Investigate and reassess relevant hazards and controls. |
| Periodic review | Confirm assessment remains suitable for the current lifting phase. |

# 15. Emergency and Recovery

A lifting emergency may involve dropped loads, crane instability, equipment failure, contact with utilities, collision or trapped personnel. The lifting team must have a clear stop-work and emergency response process. Personnel must not enter an unsafe suspended-load or crane-failure zone until the area has been made safe by competent personnel.

| Emergency Element | Requirement |
| --- | --- |
| Emergency stop | Stop lifting movement when unsafe conditions or loss of communication occurs. |
| Dropped load | Keep personnel clear, isolate the area and establish a competent recovery plan. |
| Crane instability | Stop operations, maintain exclusion and follow the approved emergency/recovery procedure. |
| Electrical contact | Keep personnel away, do not touch equipment unless isolation is confirmed, and activate emergency response. |
| Communication | Maintain reliable emergency communication and access for responders. |
| Post-incident | Provide first aid, report the incident, preserve evidence where appropriate and review controls. |

# 16. Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Project / Construction Manager | Provide resources and ensure lifting controls are implemented. |
| Lifting Manager / Supervisor | Plan, coordinate and supervise lifting operations and verify controls. |
| Lift Planner / Engineer | Prepare suitable lifting studies/plans and verify technical requirements where required. |
| Crane Operator | Operate equipment safely within approved capacity and manufacturer requirements. |
| Rigger / Slinger | Inspect and connect loads using correct rigging methods. |
| Signalman / Banksman | Provide clear signals and control the lifting interface. |
| HSE Team | Monitor compliance, advise on controls and verify site arrangements. |
| Workers / Other Personnel | Stay outside exclusion zones and follow lifting instructions. |
| Contractors / Subcontractors | Comply with lifting requirements and provide task-specific assessments where required. |

# 17. Worker Briefing / Acknowledgement

Personnel involved in or affected by lifting operations should be briefed on the lift plan, roles, communication method, exclusion zone, load path, landing area, overhead hazards, emergency arrangements and stop-work requirements before the lift.

| No. | Name | Position | Company | Signature | Date |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

# 18. Review and Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |

# 19. Disclaimer

This document is a general completed reference risk assessment for lifting operations. It is not a substitute for a project-specific lift plan, lifting study, engineered lifting assessment, equipment certification, manufacturer instructions or task-specific risk assessment. A competent person must review and adapt the assessment to actual load characteristics, crane configuration, lifting accessories, ground conditions, environmental conditions, workforce competency, client requirements and applicable requirements. Complex or critical lifts may require specialist engineering and additional controls. Risk ratings must be verified against actual conditions and controls must be monitored throughout the operation.