PROJECT NAME

WORKING AT HEIGHT
RISK ASSESSMENT

Comprehensive Risk Assessment for Work at Height Activities

| Document Information | Details |
| --- | --- |
| Resource Category | Risk Assessments |
| Document Type | Completed Practical Risk Assessment |
| Revision | Rev. 00 |
| Status | Reference Document |

Prepared as a practical HSE reference resource.

# 1. Document / Assessment Information

| Field | Details |
| --- | --- |
| Project / Site Name | ________________________________ |
| Location | ________________________________ |
| Client | ________________________________ |
| Main Contractor | ________________________________ |
| Subcontractor | ________________________________ |
| Assessment Reference | ________________________________ |
| Assessment Date | ________________________________ |
| Review Date | ________________________________ |
| Activity | Working at Height |
| Assessed By | ________________________________ |
| Position | ________________________________ |
| Reviewed By | ________________________________ |
| Approved By | ________________________________ |
| Persons at Risk | Workers, supervisors, contractors, visitors and others who may be exposed |
| Revision | 00 |
| Document Status | Completed Reference Assessment |

# 2. Purpose

This assessment identifies foreseeable hazards associated with working at height, evaluates the potential risks, and establishes practical control measures to prevent falls, reduce exposure to falling objects, and protect personnel involved in or affected by elevated work. It is a completed practical reference assessment intended to support planning, supervision, briefing and field verification. It does not replace a project- or task-specific assessment.

# 3. Scope

The assessment covers general work-at-height activities and interfaces including roof work, scaffolds, mobile scaffold towers, ladders and step ladders, temporary platforms, MEWPs, edge protection, floor and wall openings, fragile surfaces, leading edges, fall-restraint and fall-arrest systems, anchor points and lifelines, access and egress, falling tools and materials, material handling at height, weather conditions, and rescue following a fall.

# 4. Persons at Risk

| No. | Person / Group | Potential Exposure |
| --- | --- | --- |
| 1 | Workers at height | Falls, falling objects and unsafe access |
| 2 | Scaffolders | Falls during erection, alteration, use or dismantling |
| 3 | Roof workers | Falls from edges or through fragile surfaces |
| 4 | MEWP operators | Falls, entrapment, overturning or equipment-related incidents |
| 5 | Supervisors / Engineers / HSE personnel | Exposure during inspection or supervision |
| 6 | Workers below | Falling tools, materials or debris |
| 7 | Contractors / Subcontractors | Exposure during concurrent work |
| 8 | Visitors / Delivery personnel | Unplanned exposure to elevated work areas |
| 9 | Emergency / Rescue personnel | Exposure during rescue |
| 10 | Other site personnel | Exposure to work-at-height hazards |

# 5. Work at Height Control Principles

The preferred approach is to avoid work at height where reasonably practicable. Where it cannot be avoided, prevent falls through collective protection before relying on individual systems. Where a fall risk remains, minimize the consequences through suitable restraint, fall-arrest, safety nets or other appropriate systems. Controls should follow the hierarchy of controls and be selected for the task, environment and duration.

| Priority | Principle | Examples |
| --- | --- | --- |
| 1 | Avoid | Complete work at ground level where reasonably practicable. |
| 2 | Prevent | Guardrails, edge protection, compliant scaffolds and safe platforms. |
| 3 | Minimize consequences | Restraint, fall arrest, safety nets and suitable rescue arrangements. |

# 6. General Control Requirements

| No. | Control Area | General Requirement |
| --- | --- | --- |
| 1 | Planning | Plan the work, sequence, access, protection and rescue arrangements before starting. |
| 2 | Competency | Use trained and competent personnel for safety-critical work. |
| 3 | Risk Assessment | Assess task-specific hazards and changing conditions. |
| 4 | Method Statement | Provide a suitable method statement where required. |
| 5 | Access | Provide safe, suitable and maintained access and egress. |
| 6 | Equipment Selection | Select equipment appropriate to height, task, duration and environment. |
| 7 | Inspection | Inspect access and fall-protection equipment before use and at required intervals. |
| 8 | Edge Protection | Protect open edges with suitable collective protection. |
| 9 | Openings | Securely cover or guard floor, roof and wall openings. |
| 10 | Falling Objects | Prevent tools, materials and debris from falling. |
| 11 | Exclusion Zones | Control areas below and around elevated work. |
| 12 | Weather | Monitor wind, rain, heat, dust and visibility conditions. |
| 13 | PPE | Provide task-specific PPE and fall-protection equipment where required. |
| 14 | Supervision | Provide adequate supervision proportional to risk and competency. |
| 15 | Rescue | Establish a suitable rescue plan before using fall-arrest systems. |

# 7. Equipment and Systems Covered

The assessment considers commonly used work-at-height equipment and protection systems, including scaffolds, mobile towers, ladders, step ladders, temporary platforms, MEWPs, guardrails, edge protection, safety nets, fall-restraint systems, fall-arrest systems, harnesses, lanyards, lifelines, anchor points, access stairs and roof-access systems. Equipment must be selected and used according to the task and assessed conditions.

# 8. Risk Assessment Methodology

Risk is assessed using a 5 × 5 matrix. The risk score is calculated as Likelihood × Severity. Initial risk represents the exposure before additional controls are applied; residual risk represents the expected risk after the specified controls are implemented and verified.

| Likelihood | Description |
| --- | --- |
| 1 | Rare |
| 2 | Unlikely |
| 3 | Possible |
| 4 | Likely |
| 5 | Almost Certain |

| Severity | Description |
| --- | --- |
| 1 | Minor injury |
| 2 | First-aid injury |
| 3 | Medical treatment injury |
| 4 | Serious / major injury |
| 5 | Fatality / catastrophic consequence |

# 9. Risk Classification

| Score | Risk Level | General Action |
| --- | --- | --- |
| 1–4 | Low | Maintain controls and monitor. |
| 5–9 | Medium | Improve controls where reasonably practicable. |
| 10–16 | High | Immediate management attention and additional controls required. |
| 17–25 | Very High | Work should not proceed until adequate controls reduce the risk. |

# 10. 5 × 5 Risk Matrix

| Likelihood ↓ / Severity → | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 Almost Certain | 5 | 10 | 15 | 20 | 25 |
| 4 Likely | 4 | 8 | 12 | 16 | 20 |
| 3 Possible | 3 | 6 | 9 | 12 | 15 |
| 2 Unlikely | 2 | 4 | 6 | 8 | 10 |
| 1 Rare | 1 | 2 | 3 | 4 | 5 |

# 11. Detailed Working at Height Risk Assessment

The following assessment addresses representative work-at-height hazards. The risk ratings are reference values and must be verified against actual site conditions, work methods, equipment and workforce before implementation.

| No. | Activity / Hazard | Hazard Description | Persons at Risk | Potential Consequences | Existing Control Measures | Initial L | Initial S | Initial Score | Initial Level | Further Controls | Residual L | Residual S | Residual Score | Residual Level | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | General work at height | Unprotected or inadequately controlled elevated work | Workers, supervisors, contractors | Fall from height; serious injury or fatality | Plan work; avoid height where possible; provide suitable access, edge protection and supervision. | 4 | 5 | 20 | Very High | Use collective protection first; inspect controls; brief workers; restrict access; verify safe method before work. | 2 | 5 | 10 | High | Construction Manager / HSE |
| 2 | Open edges / leading edges | Work near unprotected slab, roof or platform edges | Workers, supervisors | Fall to lower level; fatal injury | Guardrails and barricades where provided; warning signs; supervision. | 4 | 5 | 20 | Very High | Install compliant guardrails/edge protection; provide toe boards; maintain exclusion zones; inspect before use. | 1 | 5 | 5 | Medium | Site Supervisor / HSE |
| 3 | Floor openings | Unprotected or inadequately covered floor openings | Workers, contractors, visitors | Fall through opening; serious injury or fatality | Covers and barricades where available; site inspections. | 4 | 5 | 20 | Very High | Secure load-capable covers or guardrails; label openings; prevent displacement; inspect daily and after changes. | 1 | 5 | 5 | Medium | Supervisor / HSE |
| 4 | Wall openings | Unprotected window, shaft or wall openings | Workers, contractors | Fall from opening | Temporary barriers and warning signs. | 4 | 5 | 20 | Very High | Install rigid guardrails or suitable protection; maintain until permanent protection is installed. | 1 | 5 | 5 | Medium | Supervisor |
| 5 | Roof work | Falls from roof edges or access points | Roof workers, supervisors | Serious injury or fatality | Access control, edge protection and fall protection where required. | 4 | 5 | 20 | Very High | Use safe roof access, edge protection and task-specific fall protection; establish rescue plan; monitor conditions. | 1 | 5 | 5 | Medium | Roof Supervisor / HSE |
| 6 | Fragile surfaces | Worker steps onto fragile roof or surface | Roof workers, contractors | Fall through surface; fatality | Warning signs and restricted access where identified. | 4 | 5 | 20 | Very High | Identify fragile areas before work; provide load-spreading platforms, guardrails or other suitable protection; prohibit unsupported access. | 1 | 5 | 5 | Medium | Roof Supervisor |
| 7 | Scaffold erection | Falls during scaffold erection or alteration | Scaffolders | Fall from height; dropped components | Competent scaffold team; basic PPE and supervision. | 4 | 5 | 20 | Very High | Use competent scaffolders, progressive edge protection, safe access, exclusion zone and inspection/tagging arrangements. | 1 | 5 | 5 | Medium | Scaffold Supervisor |
| 8 | Scaffold alteration | Unauthorized or unsafe modification of scaffold | Scaffold users, scaffolders | Collapse or fall from height | Scaffold inspection and controlled access. | 4 | 5 | 20 | Very High | Only authorized competent persons alter scaffolds; re-inspect after modification; maintain status tagging. | 1 | 5 | 5 | Medium | Scaffold Supervisor |
| 9 | Scaffold dismantling | Falls and dropped components during dismantling | Scaffolders, persons below | Falls, struck-by injury, fatality | Competent scaffolders and exclusion area. | 4 | 5 | 20 | Very High | Use planned sequence, progressive protection, controlled lowering, exclusion zone and dedicated supervision. | 1 | 5 | 5 | Medium | Scaffold Supervisor |
| 10 | Scaffold use | Unsafe access, missing components or overloading | Scaffold users | Fall, collapse or falling materials | Inspection and access ladder/stairs where provided. | 4 | 5 | 20 | Very High | Verify complete platform, guardrails, toe boards, access, load limits and current inspection status before use. | 1 | 5 | 5 | Medium | Supervisor / Scaffold Inspector |
| 11 | Mobile scaffold tower | Tower movement, overturning or unsafe access | Workers, nearby personnel | Fall or struck-by injury | Lockable casters, basic assembly controls. | 4 | 5 | 20 | Very High | Use on level ground; lock wheels; do not move with people/materials on platform; install required guardrails/outriggers; inspect before use. | 1 | 5 | 5 | Medium | Supervisor |
| 12 | Ladders | Incorrect setup, overreaching or unstable ladder | Workers | Fall from ladder | Pre-use checks and basic three-point contact instruction. | 4 | 4 | 16 | High | Use ladders only where suitable; secure correctly; maintain three-point contact; prevent overreaching; use alternatives for prolonged/high-risk work. | 2 | 4 | 8 | Medium | Supervisor |
| 13 | Step ladders | Overreaching, standing on prohibited steps or unstable setup | Workers | Fall and serious injury | Pre-use inspection and stable placement. | 3 | 4 | 12 | High | Use fully opened/locked step ladder on firm level surface; maintain safe working position; prohibit unsafe top-step use. | 1 | 4 | 4 | Low | Supervisor |
| 14 | Temporary working platform | Inadequate platform, missing guardrails or unstable support | Workers | Fall or platform failure | Platform constructed/installed by competent persons. | 4 | 5 | 20 | Very High | Verify design/installation, load capacity, full decking, guardrails and toe boards; inspect before use. | 1 | 5 | 5 | Medium | Construction Supervisor |
| 15 | MEWP use | Fall, entrapment, collision or overturning | MEWP operators, nearby workers | Serious injury or fatality | Trained operators, pre-use inspection and basic exclusion zone. | 4 | 5 | 20 | Very High | Use authorized operators; inspect ground conditions, alarms and emergency lowering; use harness/restraint where required; segregate pedestrians; follow manufacturer limits. | 1 | 5 | 5 | Medium | MEWP Supervisor |
| 16 | Fall restraint system | Incorrect setup or insufficient restraint distance | Workers | Worker reaches fall edge | Harness/lanyard available; basic instruction. | 3 | 5 | 15 | High | Verify anchor suitability and restraint length; inspect equipment; prevent access to fall edge; ensure system is compatible and correctly fitted. | 1 | 5 | 5 | Medium | Competent Person / HSE |
| 17 | Fall arrest system | Fall occurs due to incorrect connection or inadequate clearance | Workers | Arrested fall, suspension trauma, fatality | Harness and lanyard provided; anchor points identified. | 4 | 5 | 20 | Very High | Use suitable certified system; calculate clearance; inspect connectors and anchors; ensure prompt rescue plan; prevent swing-fall hazards. | 1 | 5 | 5 | Medium | Competent Person / HSE |
| 18 | Harness and lanyard | Damaged, incompatible or incorrectly fitted equipment | Workers | Equipment failure or fall | Pre-use checks and storage controls. | 3 | 5 | 15 | High | Inspect before use; quarantine damaged equipment; ensure compatibility, correct fitting and suitable anchorage; maintain inspection records. | 1 | 5 | 5 | Medium | Competent Person |
| 19 | Anchor points / lifelines | Unsuitable, damaged or incorrectly installed anchorage | Workers | System failure and fall | Designated anchors where available. | 4 | 5 | 20 | Very High | Use verified suitable anchorages; inspect installation and condition; prevent unauthorized modification; ensure system is compatible and adequately rated. | 1 | 5 | 5 | Medium | Competent Person / Engineer |
| 20 | Falling tools and materials | Tools, components or debris dropped from height | Workers below, visitors, public | Head injury, struck-by injury, fatality | Basic housekeeping and PPE. | 4 | 5 | 20 | Very High | Use toe boards, tool lanyards where appropriate, secure materials, debris controls and exclusion zones below. | 1 | 5 | 5 | Medium | Supervisor / HSE |
| 21 | Material handling at height | Manual movement of materials near edges | Workers, persons below | Dropped materials, falls, strains | Basic lifting practices and edge barriers. | 3 | 5 | 15 | High | Plan material routes; use mechanical aids; maintain edge protection; secure loads; use controlled lifting arrangements and exclusion zones. | 1 | 5 | 5 | Medium | Supervisor |
| 22 | Work near leading edges | Temporary work activity close to fall edge | Workers, contractors | Fall to lower level | Barricades and supervision. | 4 | 5 | 20 | Very High | Maintain physical edge protection; use restraint where appropriate; minimize exposure time; establish controlled work zone. | 1 | 5 | 5 | Medium | Supervisor / HSE |
| 23 | Adverse weather | Wind, rain, dust, heat or poor visibility affects elevated work | Workers, operators | Loss of balance, equipment instability, heat illness | Weather monitoring and general site controls. | 4 | 5 | 20 | Very High | Set weather limits based on equipment/task; stop work when conditions become unsafe; provide hydration/rest and reassess before restart. | 2 | 5 | 10 | High | Site Manager / HSE |
| 24 | Access and egress | Unsafe climbing, blocked access or unsuitable route | Workers | Falls, slips, delayed evacuation | Access ladders/stairs and routine checks. | 3 | 4 | 12 | High | Provide secure access, clear routes, adequate lighting and handrails; prohibit improvised access. | 1 | 4 | 4 | Low | Supervisor |
| 25 | Emergency rescue after fall | Delayed rescue after fall-arrest activation | Fallen worker, rescuers | Suspension trauma, worsening injury or fatality | Emergency arrangements and communication systems. | 4 | 5 | 20 | Very High | Prepare site-specific rescue plan; provide trained rescuers/equipment; maintain emergency access and conduct drills where appropriate. | 1 | 5 | 5 | Medium | HSE Manager / Site Manager |

Risk scores have been calculated as Likelihood × Severity. Residual ratings assume the listed additional controls are implemented and verified.

# 12. Key Working at Height Safety Requirements

| No. | Requirement | Expected Practice |
| --- | --- | --- |
| 1 | Planning | Plan work, access, protection, sequencing and rescue before starting. |
| 2 | Competency | Use competent workers and operators for safety-critical activities. |
| 3 | Risk Assessment | Review hazards and controls before and during changing work. |
| 4 | Method Statement | Use task-specific method statements where required. |
| 5 | Access | Provide safe, suitable and maintained access and egress. |
| 6 | Scaffolds | Use inspected, suitable scaffolds with required components and access. |
| 7 | Ladders | Use only where suitable; secure and maintain safe working position. |
| 8 | MEWPs | Use authorized operators, pre-use checks, suitable ground and required fall protection. |
| 9 | Edge Protection | Provide effective collective protection at exposed edges. |
| 10 | Openings | Securely cover or guard openings and prevent displacement. |
| 11 | Fall Protection | Select restraint or arrest systems based on assessed need and compatibility. |
| 12 | Inspection | Inspect equipment before use and at required intervals. |
| 13 | Falling Objects | Secure tools/materials and establish exclusion zones. |
| 14 | Weather | Monitor conditions and stop elevated work when unsafe. |
| 15 | Rescue | Maintain a suitable rescue plan and competent response arrangements. |
| 16 | Supervision | Provide supervision proportionate to risk and competency. |
| 17 | PPE | Provide and use task-specific PPE. |

# 13. Inspection and Monitoring

Work-at-height arrangements should be subject to pre-use checks, routine inspections and field verification. Scaffolds, access equipment, MEWPs, harnesses, lanyards, anchor systems and other fall-protection equipment should be inspected by competent persons at the required frequency and after events that could affect integrity. Defective equipment must be removed from service, identified and controlled until repaired or replaced.

| Inspection / Monitoring Activity | Minimum Focus |
| --- | --- |
| Pre-use checks | Condition, setup, guards, access, connectors, alarms and visible defects. |
| Scaffold inspection | Structure, platforms, guardrails, toe boards, access, foundations and status. |
| Fall-protection inspection | Harnesses, lanyards, connectors, anchors, lifelines and compatibility. |
| MEWP inspection | Controls, alarms, emergency systems, platform, tires/outriggers and ground conditions. |
| Site HSE inspection | Edges, openings, exclusion zones, housekeeping and changing conditions. |
| Supervisor monitoring | Work method, worker behavior, access, protection and compliance. |
| Post-change verification | Reassess and inspect after alteration, relocation, weather or incident. |

# 14. Monitoring and Review of the Risk Assessment

| Review Trigger | Required Action |
| --- | --- |
| Before work starts | Confirm hazards, controls, access and rescue arrangements. |
| Change in work method | Review affected hazards and controls before proceeding. |
| Change in location or site conditions | Reassess edges, openings, access, ground and interfaces. |
| New equipment | Confirm suitability, competency, inspection and operating limits. |
| New hazard identified | Stop or control the affected work and update the assessment. |
| Incident / near miss | Investigate and reassess relevant hazards and controls. |
| Significant weather change | Reassess elevated work and equipment operating conditions. |
| Change in workforce competency | Verify training and supervision requirements. |
| Periodic review | Review continued suitability and effectiveness of controls. |

# 15. Emergency and Rescue

A suitable rescue arrangement should be established before work starts where fall-arrest systems are used. The plan should consider communication, access, competent rescuers, rescue equipment, emergency routes, prompt recovery of a suspended person, first aid and escalation. Rescue arrangements should be realistic for the actual location and should not rely solely on external emergency services.

| Emergency Element | Requirement |
| --- | --- |
| Communication | Provide reliable means to raise an emergency. |
| Rescue Plan | Define the rescue method, equipment, roles and access. |
| Competent Rescuers | Identify personnel trained and capable of carrying out the planned rescue. |
| Rescue Equipment | Keep required equipment available, accessible and maintained. |
| Emergency Access | Keep routes and access points clear for response. |
| Post-Fall Response | Provide first aid, preserve the scene where appropriate and report the incident. |

# 16. Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Project / Construction Manager | Provide resources and ensure implementation of controls. |
| HSE Manager / HSE Team | Review, monitor and verify controls; provide advice and assurance. |
| Supervisors | Implement controls, brief workers and monitor work. |
| Competent Persons | Inspect and assess relevant equipment and protection systems. |
| Workers | Follow procedures, use equipment correctly and report hazards/defects. |
| Contractors / Subcontractors | Comply with requirements and provide task-specific assessments where required. |
| Scaffold Team | Erect, alter and dismantle scaffolds safely and under competent supervision. |
| MEWP Operators | Conduct checks, operate equipment safely and follow manufacturer requirements. |

# 17. Worker Briefing / Acknowledgement

Personnel affected by the assessment should be briefed on the relevant hazards, controls, work method, access arrangements, exclusion zones, PPE and emergency arrangements before starting work. The following record may be used to document the briefing.

| No. | Name | Position | Company | Signature | Date |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

# 18. Review and Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |

# 19. Disclaimer

This document is a general completed reference risk assessment for working at height. It is not a substitute for a project-specific or task-specific risk assessment. A competent person should review and adapt the assessment before implementation to reflect actual site conditions, work methods, equipment, materials, workforce competency, environmental conditions, client requirements and applicable requirements. Additional task-specific assessments may be required for high-risk activities. Risk ratings must be verified against actual conditions and the effectiveness of controls must be monitored throughout the work.